- Link to each section

Description									 2

- Limitations of the Prioritization Tool						 4

- Application of the Prioritization Tool						 5

Department Prioritization Tools

Dialysis										 6

- Environmental Services								 7

- Facilities Maintenance/Engineering						11

- Laboratory/Pathology/Histology							20

- Nutrition Services								26

- Pharmacy									34

- Radiology									39

- Sterile Processing								41

- Surgical Services									43

- DESCRIPTION

Hospitals for a Healthy Environment (H2E) recognized a number of factors in developing the Hazardous Chemical Waste Prioritization Tool.  Regulatory burden, employee health and safety, the long-lived toxicity of PBT chemicals and volume are the factors used.  These factors are further defined below.

Regulatory burden: Hazardous chemicals are subject to a variety of regulations when discarded.  The chemicals that are regulated under EPA’s Resource Conservation and Recovery Act (RCRA) are one set of criteria used.  RCRA “P” listed chemicals, due to their acute toxicity and increased regulatory burden were considered in an additional category.

If the chemical is considered an acutely toxic pesticide, Category I by the US EPA Office of Pesticide Programs, it then is also a hazardous waste when disposed of.   See www.epa.gov/oppfead1/labeling/lrm/chap-08.htm for further information.

One point was given if the chemical is a hazardous waste.  An additional point was given if the waste is “P” listed.  RCRA listed hazardous wastes and characteristics can be found in Appendix A.  More detailed information can be found on EPA’s web site at www.epa.gov/epaoswer/hazwaste/state/cfrlist.htm.

Employee Health and Safety: the Occupational Safety and Health Administration’s (OSHA) hazardous substance lists defined this criterion.  The lists used were the OSHA Pharmaceuticals List (OSHA Instruction CPL 2-2.20B CH-4) (Appendix D) and the Air Contaminants list of 29 CFR 1910.1000 subpart Z (Appendix C).

Note:  the presence of a chemical on a material safety data sheet (MSDS) indicates it is a “hazardous substance” per OSHA criteria; hence presence on the “Air Contaminants” list presents another hazard.  One point was given if the waste is listed on OSHA’s hazardous substance list.

Persistent, Bioaccumlative and/or Toxic Chemicals (PBTs): Discussed on page 10 of the Chemical Minimization Tool, PBTs were identified using the draft EPA-derived list of 53 chemicals targeted for elimination under the National Waste Minimization Plan.   The full list of PBTs can be found in Appendix E.  One point was given if the waste is a PBT.

Volume:  Chemicals may present more of a concern by the nature of a larger volume being used and ultimately disposed of.  The H2E tool attempts to identify those large volume chemicals.  H2E acknowledges that volumes of chemicals used will vary with institution size.  However, the amounts of chemicals used within an individual institution will be proportionate.  For example, most institutions will use relatively large volumes of floor stripper if they have hard floors, but use considerably lower volumes of furniture polish.

For the purpose of gauging, volumes have been broken down as follows: Large > 100 gal/year, Medium 50-100 gal/year; Small 15-50 gal/year; Very Small <15 gal/year.  1 point for Large or Med. Volume, 0.5 point for small, 0 points for Very Small.  Exception: for pharmaceuticals where “large volume” was based on frequency of use.

Total: Each factor present for a given chemical was given one point (volume points given on basis listed above).  If the chemical scored 3 or greater it was identified as a “priority chemical”.

- OTHER CRITERIA

Pesticides:  Include antimicrobials and disinfectants.  This class of chemicals is of special concern as they are designed to kill.  Some pesticides such as Lindane are included as hazardous chemicals under RCRA, many others though, are not.  However, if a pesticide is labeled “acutely toxic” it must be managed as a hazardous waste.  Pesticides are used throughout healthcare, especially in the form of disinfectants and sterilants.

Pesticide use should be thoroughly assessed within each department.  The proper pesticide and its application should be reviewed.  Employees must be trained on proper use.  Knowledge of pesticides and their relative toxicity can be assessed using “Leachability” criteria and label review.  Choose pesticides with the lowest leachability factor possible that meets your need.  Pesticides with lower leachability stay in the soil allowing them to be effective and reducing the amount that contaminates ground or surface water.  Implement integrated pest management practices.

Aquatic Toxicity: In some cases, chemicals generated at a healthcare facility are poured down the sewer.  Most chemicals are not adequately treated at wastewater treatment plants and hence discharged to surface waters such as lakes, rivers and oceans.  Many of these sewered chemicals may become regulated as the Clean Water Act provisions are effected.  The Aquatic Toxicity chemical list is an EPA-derived list found at www.epa.gov/ecotox.  The database allows you to retrieve information by CAS number.

- LIMITATIONS OF THE PRIORITIZATION TOOL

The number of chemicals in use today has been estimated at 80,000!  The number of chemicals used in healthcare ranks in the thousands as well.  H2E recognizes that not all hazardous aspects of chemicals are identified using this tool.  Also not all chemicals used in healthcare have had the tool applied to them

In part, due to the surfeit of chemicals, not all are regulated. New chemicals enter the market and workplace much more rapidly than regulatory agencies can test and assess them. Hence, although a chemical may not be technically regulated, such as some of the newer pesticides or disinfectants by OSHA or RCRA, it is important that its hazards be recognized.  Review of a product’s material safety data sheet (MSDS) can help identify the steps necessary to ensure both occupational and environmental safety.

- Carcinogens are another category of chemicals not generally identified using the H2E Chemical Minimization Prioritization Tool.  The tool is designed to identify those chemicals that present an immediate or acute hazard.  Carcinogens frequently present a hazard associated with chronic use; hence they are not usually covered by the regulations used for this tool.  Additionally, many

- carcinogens have been identified only recently and the regulatory agencies have not yet had the opportunity to include them in the rules.

- The tool has been applied using lists of chemicals from a variety of healthcare institutions.  H2E believes the chemical lists are representative, that is, most healthcare institutions will be engaged in similar processes.  However, the exact chemicals or combination of products will vary from institution to institution.  Hence, the prioritized chemical lists are designed as guidance to assist healthcare organizations with identifying those chemicals that present significant concerns.

- APPLICATION OF THE PRIORITIZATION TOOL

H2E identified a number of chemicals used in healthcare.  Each chemical was then cross-referenced against the criteria listed above.  For each match, the chemical was given a point.  Those chemicals with a score of 3 or higher have been targeted as “Priority” Chemicals to be reduced or eliminated.

- An example is given below.

| Chemical Name | Common or Trade Name | CAS Number | OSHA | PBT | RCRA CODE | P-List RCRA | Volume | TOTAL |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Acetic acid |  | 64-19-7 | 1 | 0 | D002 | 0 | 0 | 2 |
| Acetone |  | 67-64-1 | 1 | 0 | U200, F003 | 0 | 0 | 2 |
| Acetophenyl-mercury | Phenylmercuric acetate | 62-38-4 | 0 or 1 (Hg salt) | 1 | P092, D009 | 1 | 0 | 3-4 |

Results:

Generally those chemicals that are PBTs or are found on the RCRA P-list were those that scored highest using the H2E Tool.  The chemicals that scored 3 or more, the departments where they may be found, and their typical use are listed in Table 3 of the Chemical Minimization Chapter.  Minimization tips for priority chemicals may be found in the individual department areas of the main chapter.  Priority Chemicals may be found in other departments, but generally not in significant volumes.

The Prioritization Tools for each department follow.

DIALYSIS Prioritization Tool

| PRODUCT NAME | INGREDIENTS | CAS NUMBER | OSHA | PBT | VOLUME | RCRA | P LIST | TOTAL |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Formula 409 Antibacterial All Purpose Cleaner | Ethylene Glycol, Monobutyl, Ether | 111-76-2 | 1 | 0 | 0 | 1 ?PEST | 0 | 2 |
| ExSept ™ | Available chlorine, Sodium Chloride, Water | 7782-50-5 | 1 | 0 | 0 | 0 | 0 | 1 |
| TDS/Conductivity Standard Solution 442 Solution | Sodium Bicarbonate Sodium Sulfate Sodium Chloride | 144-55-8 7757-82-6 7647-14-5 | 0 0 0 | 0 0 0 | 0 0 0 | 0 0 0 | 0 0 0 | 0 0 0 |
| Septisol Foam | Ethyl Alcohol Hexachlorophene | 64-17-5 70-30-4 | 1 0 | 0 0 | 0 0 | D001 U132 | 0 0 | 2 1 |
| Renalin® Cold Sterilant | Hydrogen Peroxide 20% Peracetic Acid Acetic Acid | 7722-84-1 79-21-0 64-19-7 | 1 0 1 | 0 0 0 | 1 1 1 | D001 D002 D002 | 0 0 0 | 3 2 3 |
| Povidone Iodine Solution, U.S.P. | (1-(2-OxO-1-pyrrolidinyl) iodine | 25655-41-8 | 0 | 0 | 0 | 0 | 0 | 0 |
| Isopropyl Alcohol (70%) | Isopropyl Alcohol | 67-63-0 | 1 | 0 | 0 | D001 | 0 | 2 |
| Hydrogen Peroxide 3% Solution | Hydrogen Peroxide | 7722-84-1 | 1 | 0 | 0 | D001 | 0 | 2 |
| Hemoccult Developer | Ethyl Alcohol Methanol (Denaturant) Hydrogen Peroxide | 64-17-5 67-56-1 7722-84-1 | 1 1 1 | 0 0 0 | 0 0 0 | D001 D001 D003 | 0 0 0 | 2 2 2 |
| Bleach | Sodium Hypochlorite 5.25% | 7681-52-9 | 0 | 0 | 1 | D003 | 0 | 2 |
| Formalin | Formaldehyde Methanol | 67-56-1 | 1 1 | 0 0 | 1 1 | U122 U154 | 0 0 | 3 3 |
| Glutaraldehyde (Cidex™) |  |  | 1 | 0 | 0.5 | PEST | 0 | 3.5 |
| DEHP | Di (ethylhexyl) phthalate | 117-81-7 | 1 | 1 | 1 | U028 | 0 | 4 |
| Acetic Acid Aqueous Solution | Acetic Acid, Glacial | 64-19-7 | 1 | 0 | 1 | 1 if pH <=2.5 D002 | 0 | 2-3 |

ENVIRONMENTAL SERVICES Prioritization Tool

| Chemical Name | Common, Trade Name or Use | CAS Number | OSHA | PBT | RCRA Code | P RCRA |  | Volume | TOTAL |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Acetic acid |  | 64-19-7 | 1 | 0 | D002 | 0 |  | 0 | 2 |  |
| Acetone | nail polish remover, paint thinner | 67-64-1 | 1 | 0 | D001 F003 U002 | 0 |  | 0 | 2 |  |
| Acetophenylmercury | Phenylmercuric acetate, preservative in paints, fungicides | 62-38-4 | 0 | 1 | 1 | P092 D009 | 0 |  | 3 |  |
| Acrylonitrile |  | 107-13-1 | 1 | 0 | U003 D001 | 0 |  | 0 | 2 |  |
| n-alkyl dimethyl benzyl ammonium chloride | "Quat" disinfectant | 8001-54-5 | 0 | 0 | PEST | 0 |  | 1 | 2 |  |
| n-alkyl dimethylethyl benzyl ammonium chloride | "Quat" disinfectant Benzalkonium chloride | 68956-79-6 | 0 | 0 | PEST | 0 |  | 1 | 2 |  |
| Ammonium chloride | Laundry detergent | 12125-02-9 | 1 | 0 | PEST | 0 |  | 1 | 3 |  |
| Ammonium hydroxide | Ammonia/cleaner | 1336-21-6 | 1 | 0 | D002 | 0 |  | 0.5 | 1.5 |  |
| Benzene | Spot remover | 71-43-2 | 1 | 0 | U019 D001 | 0 |  | 0 | 2 |  |
| Benzalkonium chloride | Quat" disinfectant |  | 0 | 0 | PEST | 0 |  | 1 | 2 |  |
| Benzoic acid | Retardex preservative | 65-85-0 | 0 | 0 | 0 | 0 |  | 0 | 0 |  |
| Benzothiazolethiol, 2- | Captax, MBT | 149-30-4 | 0 | 0 | PEST | 0 |  | 0 | 1 |  |
| Benzoyloxytributyl-stannate | Tributyltin benzoate, floor cleaner | 4342-36-3 | 1 | 0 | 0 | 0 |  | 1 | 2 |  |
| Benzyl Alcohol |  | 100-51-6 | 0 | 0 | 0 | 0 |  | 0 | 0 |  |
| Biphenylol, sodium salt, 2- | Bactrol, Mil-du-rid, Dowcide | 132-27-4 | 0 | 0 | PEST | 0 |  | 1 | 2 |  |
| Butyl acetate | n-butyl acetate | 123-86-4 | 1 | 0 | D001 | 0 |  | 0 | 2 |  |
| n-Butyl alcohol | Butanol | 71-36-3 | 1 | 0 | U031 D001 | 0 |  | 0 | 2 |  |
| n-butyl phthalate | plasticizer floor finishes | 84-74-2 | 1 | 1 | U069 | 0 |  | 1 | 4 |  |
| Carbon tetrachloride | spot remover | 56-23-5 | 1 | 0 | U211 F002 | 0 |  | 0 | 2 |  |
| Chloroform | spot remover | 67-66-3 | 1 | 1 | D022 U044 | 0 |  | 0 | 3 |  |
| Chromium (VI) oxide |  | 1333-82-0 | 1 | 1 | D007 | 0 |  | 0 | 3 |  |
| Citric acid |  | 77-92-9 | 0 | 0 | 0 | 0 |  | 0 | 0 |  |
| Copper (II) sulfate (1:1) | Blue copper, fungicide | 7758-98-7 | 0 | 1 | PEST | 0 |  | 1 | 3 |  |
| Coumarin | Rattex (TM), Warfarin | 91-64-5 | 1 | 0 | P001 | 1 |  | 0 | 3 |  |
| Cyclohexanol | Paints, pesticides | 108-93-0 | 1 | 0 | D001 | 0 |  | 0.5 | 2.5 |  |
| di-n-alkyl dimethyl ammonium chloride | quat disinfectant |  | 0 | 0 | PEST | 0 |  | 1 | 2 |  |
| Diazinon | Organophosphate pesticide | 333-41-5 | 1 | 0 | PEST | 0 |  | 0 | 2 |  |
| Dichloromethane | Methylene chloride, paint stripper | 75-09-2 | 1 | 0 | F002 | 0 |  | 0.5 | 2.5 |  |
| Dichlorvos(2,2-dichlorovinyl dimethyl ester phosphoric acid) | pesticide, No Pest Strip | 62-73-7 | 1 | 0 | PEST | 0 |  | 0 | 2 |  |
| Diethylene glycol monobutyl ether |  | 112-34-5 | 0 | 0 | 0 | 0 |  | 1 | 1 |  |
| Dioxane, 1,4- Diethylene dioxide | wax & resin solvent | 123-91-1 | 1 | 0 | U108 D001 | 0 |  | 0 | 2 |  |
| Dipropylene glycol methyl ether | Arcosolve, cleaner | 34590-94-8 | 1 | 0 | 0 | 0 |  | 1 | 2 |  |
| Ethanol, 2-butoxy- | BUCS, cleaner | 111-76-2 | 1 | 0 | 0 | 0 |  | 1 | 2 |  |
| Ethanol, 2-(2-ethoxyethoxy)-, acetate | Carbitol Cellosolve, degreaser | 111-90-9 | 0 | 0 | D001 | 0 |  | 1 | 2 |  |
| Ethanolamine | degreaser | 141-43-5 | 1 | 0 | D002 | 0 |  | 1 | 2 |  |
| Ethoxyethanol, 2- | Cellosolve, degreaser | 110-80-5 | 1 | 0 | U359 | 0 |  | 1 | 3 |  |
| Ethyl acrylate | adhesive | 140-88-5 | 1 | 0 | D001 U113 | 0 |  | 0 | 2 |  |
| Ethyl alcohol | Alcohol; ethanol | 64-17-5 | 1 | 0 | D001 | 0 |  | 0 | 2 |  |
| Ethylene glycol | Antifreeze, cleaner ingredient | 107-21-1 | 0 | 0 | 0 | 0 |  | 0.5 | 0.5 |  |
| Ethylene oxide | Sterilant | 75-21-8 | 1 | 0 | U115 | 0 |  | 0 | 2 |  |
| Formaldehyde | Fixative, sterilant | 50-00-0 | 1 | 0 | U122 | 0 |  | 0 | 2 |  |
| Glycerin |  | 56-81-5 | 1 | 0 | 0 | 0 |  | 0 | 1 |  |
| Glycolic acid |  |  | 0 | 0 | 0 | 0 |  | 0 | 0 |  |
| Isopropyl alcohol | Disinfectant | 67-63-0 | 1 | 0 | D001 | 0 |  | 0 | 2 |  |
| Lactic acid | Plasticizer - floor finishes | 50-21-5 | 0 | 0 | 0 | 0 |  | 0.5 | 0.5 |  |
| Maleic anhydride | Paint ingredient, resins | 108-31-6 | 1 | 0 | 1 | 0 |  | 0 | 2 |  |
| Methyl alcohol | Methanol, antifreeze, solvent | 67-56-1 | 1 | 0 | D001 F004 | 0 |  | 0.5 | 2.5 |  |
| Methyl ethyl ketone | 2-butanone, paint solvent | 78-93-3 | 1 | 0 | D001 F005 | 0 |  | 0.5 | 2.5 |  |
| Methyl methacrylate | Adhesive | 80-62-6 | 1 | 0 | D001 | 0 |  | 0 | 2 |  |
| Morpholine | Wax ingredient | 110-91-8 | 1 | 0 | D001 | 0 |  | 1 | 3 |  |
| Myristalkonium chloride | disinfectant | 139-08-2 | 0 | 0 | PEST | 0 |  | 1 | 2 |  |
| Nitrilotriethanol, 2,2',2"- |  | 102-71-6 | 0 | 0 | 0 | 0 |  | 0 | 0 |  |
| Nonylphenol |  |  | 0 | 0 | 0 | 0 |  | 0 | 0 |  |
| Oxalic acid |  | 144-62-7 | 1 | 0 | 0 | 0 |  | 0 | 1 |  |
| Pentanediol, 1,5- | Pentamethylene | 287-92-3 | 0 | 0 | 0 | 0 |  | 0 | 0 |  |
| Pentylphenol, para-tert |  | 14938-35-3 | 0 | 0 | PEST | 0 |  | 0 | 1 |  |
| Perchloroethylene | "perc", spotting agent, dry cleaning solvent | 127-18-4 | 1 | 0 | F002 | 0 |  | 0 | 2 |  |
| Phenol |  | 108-95-2 | 1 | 1 | U188 PEST | 0 |  | 0 | 3 |  |
| Phenol, 4-chloro-2-cyclopentyl- | phenolic disinfectant |  | 0 | 0 | PEST | 0 |  | 0 | 1 |  |
| 2-phenylphenol |  | 90-43-7 | 0 | 0 | 0 | 0 |  | 0 | 0 |  |
| Phosphoric acid | porcelain cleaner | 7664-38-2 | 1 | 0 | D002 | 0 |  | 1 | 3 |  |
| Potassium chloride |  | 7447-40-7 | 0 | 0 | 0 | 0 |  | 0 | 0 |  |
| Potassium hydroxide | caustic potash | 1310-58-3 | 0 | 0 | D002 | 0 |  | 0 |  | 1 |
| Propanediol, 1,2- | Antifreeze, Dowfrost | 57-55-6 | 0 | 0 | 0 | 0 |  | 0 | 0 |  |
| Propylene glycol monomethyl ether | Antifreeze, Dowanol 33B | 107-98-2 | 0 | 0 | 0 | 0 |  | 1 | 1 |  |
| Propylene oxide |  | 75-56-9 | 1 | 0 | D001 | 0 |  | 0 | 2 |  |
| Salicylic acid |  | 69-72-7 | 0 | 0 | 0 | 0 |  | 0 | 0 |  |
| Sodium carbonate | soda ash, Trona, carbonic acid | 497-19-8 | 1 | 0 | 0 | 0 |  | 0 | 1 |  |
| Sodium chloride | (common) salt | 7647-14-5 | 0 | 0 | 0 | 0 |  | 0 | 0 |  |
| Sodium dodecyclbenzene-sulfonate | dodecyclbenzenesulfonic acid sodium salt | 25155-30-0 | 0 | 0 | 0 | 0 |  | 0 | 0 |  |
| Sodium hypochlorite | bleach | 7681-52-9 | 0 | 0 | 0 | 0 |  | 1 | 1 |  |
| Sodium lauryl sulfate | shampoo/detergent additive | 151-21-3 | 0 | 0 | 0 | 0 |  | 0 | 0 |  |
| Sodium metasilicate | Metso Beads, cleaner | 6834-92-0 | 1 | 0 | 1 | 0 |  | 0 | 2 |  |
| Sodium (I) Nitrate | chile saltpeter | 7631-99-4 | 0 | 0 | 1 | 0 |  | 1 | 2 |  |
| Sodium nitrite |  | 7632-00-0 | 0 | 0 | 1 | 0 |  | 0 | 1 |  |
| Sodium xylene sulfonate |  | 1300-72-7 | 0 | 0 | 0 | 0 |  | 0 | 0 |  |
| Stearic acid | Century | 57-11-4 | 0 | 0 | 0 | 0 |  | 0 | 0 |  |
| Styrene | Cinnamenol, resin | 100-42-5 | 1 | 0 | D001 | 0 |  | 0 | 2 |  |
| Sulfuric acid | sulfuric acid | 7664-93-9 | 1 | 0 | D002 | 0 |  | 0 | 2 |  |
| Tertiary amylphenol | phenolic disinfectant | 80-46-6 | 0 | 0 | PEST | 0 |  | 1 | 2 |  |
| Toluene |  | 108-88-3 | 1 | 0 | F005 U220 | 0 |  | 0 | 2 |  |
| Tributyltin benzoite | floor cleaner | 4342-36-3 | 1 | 0 | 0 | 0 |  | 1 | 2 |  |
| Triazine-2,4,6,(1H,3H,5H)-trione, 1,3-dichloro-, potassium salt | pesticide |  | 0 | 0 | 1 | 0 |  | 0 | 1 |  |
| Trichloroethane, 1,1,1- | contact cleaner, stainless steel cleaner | 71-55-6 | 1 | 1 | F001 U227 | 0 |  | 0 | 3 |  |
| Urea | Carbamide | 57-13-6 | 0 | 0 | 0 | 0 |  | 0 | 0 |  |
| Xylene | paint thinner | 1330-20-7 | 1 | 0 | F003 U239 | 0 |  | 0 | 2 |  |
| Zinc chloride |  | 7646-85-7 | 1 | 0 | 0 | 0 |  | 0 | 1 |  |
| Zinc oxide | paint pigment | 1314-13-2 | 1 | 0 | D001 | 0 |  | 1 | 3 |  |
| Zinc sulfate |  | 7733-02-0 | 1 | 0 | D001 | 0 |  | 0 | 2 |  |

ENGINEERING/FACILITIES/MAINTENANCE Prioritization Tool

| Chemical Name & Ingredients | # Common/Trade Name or Use | CAS# | OSHA | PBT | RCRA | P? | VOL | TOTAL |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # Acetylene | welding gas | 74-86-2 | 1 | 0 | D001 | 0 | 1 | 3 |
| Acetylene |  | 74-86-2 | 1 | 0 | D001 | 0 | 1 | 3 |
| Ethane |  | 74-84-0 | 1 | 0 | D001 | 0 | 1 | 3 |
| Ethylene |  | 74-85-1 | 1 | 0 | D001 | 0 | 1 | 3 |
| Methane |  | 74-82-8 | 1 | 0 | D001 | 0 | 1 | 3 |
| Acetone |  | 67-64-1 | 1 | 0 | D001 | 0 | 1 | 3 |
| Propane |  | 74-98-6 | 1 | 0 | D001 | 0 | 1 | 3 |
| Propene |  | 115-07-1 | 1 | 0 | D001 | 0 | 1 | 3 |
| # Acid | boiler chemical |  |  |  |  |  |  |  |
| 5-Chloro-2-methyl-4-isothiazolin-3-one |  | 26172-55-4 | * |  |  |  |  | * |
| 2-Methyl-4-isothiazolin-3-one |  | 2682-20-4 | * |  |  |  |  | * |
| Nitrate Compounds 2% |  |  | * |  |  |  |  | * |
| # Acrylic Sealant |  |  |  |  |  |  |  |  |
| Acrylic Emulsion* |  |  | * |  |  |  |  | * |
| Aluminum Hydroxide |  | 21645-51-2 | N/A | 0 | 0 | 0 | 1 | 1 |
| Carbonic Acid, Calcium Salt |  | 00471-34-1 | N/A | 0 | 0 | 0 | 1 | 1 |
| Graphite Intercalation Compound |  | 12689-13-3 | N/A | 0 | 0 | 0 | 1 | 1 |
| Talc |  | 14807-96-6 | N/A | 0 | 0 | 0 | 1 | 1 |
| Ash |  | 68131-74-8 | N/A | 0 | 0 | 0 | 1 | 1 |
| Polyisobutylene |  | 09003-27-4 | * | 0 | 0 | 0 | 1 | *1 |
| 1,3,5- triazine - 2,4,6 – triamine |  | 00108-78-1 | 0 | 0 | 0 | 0 | 1 | 1 |
| Ethylene glycol |  | 00107-21-1 | 1 | 0 | 0 | 0 | 1 | 2 |
| Polypropylene Glycol Alkyl Phenyl Ether |  | 09064-13-5 | * | 0 | * | * | 1 | *1 |
| Sodium Hydroxide |  | 01310-73-2 | 1 | 0 | D002 | 0 | 1 | 3 |
| # Adhesive | contact cement |  |  |  |  |  |  |  |
| Hexane |  | 110-54-3 | 1 | 0 | D001 | 0 | 1 | 3 |
| Toluene |  | 108-88-3 | 1 | 0 | D001 F005 | 0 | 1 | 3 |
| Acetone |  | 67-64-1 | 1 | 0 | D001 F003 | 0 | 1 | 3 |
| # Lubricant, Aerosol | Spray Lube |  |  |  |  |  |  |  |
| Petroleum Distillate |  | 64742-47-8 | 1 MIST | 0 | D001 | 0 | 0 | 2 |
| Inhibited Paraffinic Oil |  |  | 1 MIST | 0 | 0 | 0 | 0 | 1 |
| Carbon Dioxide |  | 124-38-9 | 1 MIST | 0 | D001 | 0 | 0 | 2 |
| # Algicide |  |  |  |  |  |  |  |  |
| polyoxyethylene(dimethylamino) ethylene (dimethylamino) ethylene dichloride |  | 31075-24-8 | * |  | PEST |  |  | 1* |
| Alkyd Enamel | semi-gloss paint |  |  |  |  |  |  |  |
| Aliphatic Hydrocarbon |  | 64742-8-7 | 1 | 0 | D001 | 0 | 1 | 3 |
| # Ammonia | ammonia cleaner | 7664-41-7 | 1 | 0 | D002 | 0 | 0 | 2 |
| # Argon | Argon gas | 7440-37-1 | 0 | 0 | D001 | 0 | 1 | 2 |
| # Bromthymol Blue |  | 76-59-5 | 0 | 0 | 0 | 0 | 0 | 0 |
| # Cable Degreaser |  |  |  |  |  |  |  | 0 |
| Trichloroethane |  | 71-55-6 | 1 | 1 | F001 | 0 | 0 | 3 |
| Perchloroethylene |  | 127-18-4 | 1 | 0 | F001 | 0 | 0 | 2 |
| # Carpenter Glue |  |  |  |  |  |  |  |  |
| # Calcium Buffer Solution |  |  |  |  |  |  |  |  |
| Sodium Hydroxide |  |  | 1 | 0 | D002 | 0 | 0 | 2 |
| # Chlorine Reagent | boiler chemical; chlorine detection reagent |  |  |  |  |  |  |  |
| Salt of N,N-Diethyl-p-Phenylenediamine |  | 93-05-0 |  |  |  |  |  |  |
| Carboxylate Salt |  |  | * | * | * | * | * | * |
| Sodium Phosphate, Dibasic |  | 7558-79-4 | 0 | 0 | 0 | 0 | 0 | 0 |
| Ethylenediaminetetraacetic Acid, Disodium Salt |  | 6381-92-6 | 0 | 0 | 0 | 0 | 0 | 0 |
| # Chlorodifluoromethane | Freon 22 | 75-45-6 | 1 | 0 | F002 | 0 | 1 | 3 |
| # Cleaner |  |  |  |  |  |  |  |  |
| Phosphoric Acid |  | 2809-21-4 | 0 | 0 | D002 | 0 | 1 | 2 |
| Potassium Hydroxide |  | 1310-58-3 | 1 | 0 | D002 | 0 | 1 | 3 |
| Cleaning Compound |  |  |  |  |  |  |  |  |
| Sodium Metasilicate |  | 6834-92-0 | 0 | 0 | D002 | 0 | 0 | 1 |
| # Coating | Kure N Seal |  |  |  |  |  |  |  |
| Xylene |  | 1330-20-7 | 1 | 0 | D001 | 0 | 1 | 3 |
| Naphtha-light aromatic |  | 64742-95-6 | 1 | 0 | D001 | 0 | 1 | 3 |
| Mineral spirits |  | 8052-41-3 | 1 | 0 | D001 | 0 | 1 | 3 |
| 1,2,4 Trimethyl Benzene |  | 95-63-6 | 1 | 0 | D001 | 0 | 1 | 3 |
| # Contact Cleaner |  |  |  |  |  |  |  |  |
| Hexane |  | 110-54-3 | 1 | 0 | D001 | 0 | 0.5 | 2.5 |
| Denatured ethanol |  | 64-17-5 | 1 | 0 | D001 | 0 | 0.5 | 2.5 |
| Methanol |  | 67-56-1 | 1 | 0 | D001 | 0 | 0.5 | 2.5 |
| Carbon Dioxide |  | 124-38-9 | 1 | 0 | D001 | 0 | 0.5 | 2.5 |
| # Boiler Chemical, Acid | CC-33L (Mitco) |  |  |  |  |  |  |  |
| 5-chloro-2-Methyl-4-isothiazolin-3-one |  | 26172-55-4 | 0 | 0 | PEST | 0 | 0 | 1 |
| 2-Methyl-4-isothiazolin-3-one |  | 2682-20-4 | * | * | * | * | * | * |
| # Nitrate Compounds 2% |  |  | 0 | 0 | 0 | 0 | 0 | 0 |
| Corrosion Inhibitor |  |  |  |  |  |  |  |  |
| Sodium Nitrite |  | 7632-00-0 | DUST | 0 | D001 | 0 | 1 | 3 |
| Boric Acid, Disodium Salt, Pentahydrate |  | 12179-04-3 | DUST | 0 | 0 | 0 | 1 | 2 |
| # Cresol Red | dye Indicator |  | 0 | 0 | 0 | 0 | 0 | 0 |
| # Boiler Chemical | CW-2IL (Mitco) |  | 1 | 0 | 0 | 0 | 1 | 2 |
| Sodium Hydroxide |  | 1310-73-2 | 1 | 0 | D002 | 0 | 1 | 3 |
| Borates, Tetra, Sodium Salt, Pentahydrate |  | 1303-96-4 | 1 | 0 | D001 | 0 | 1 | 3 |
| Sodium Nitrite |  | 7632-00-0 | 0 | 0 | 0 | 0 | 1 | 1 |
| 2-Mercaptobenzothiazole (MBT) | Captax | 149-30-4 | DUST | 0 | PEST | 0 | 1 | 3 |
| # Degreaser | DGT Plus |  |  |  |  |  |  |  |
| # De-Scaling Agent | Ice-Machine cleaner |  |  |  |  |  |  |  |
| Phosphoric acid |  | 7664-38-2 | 1 | 0 | D001 | 0 | 0 | 2 |
| Citric Acid |  | 77-92-9 | 0 | 0 | 0 | 0 | 0 | 0 |
| # Drain Cleaner | Drano (TM) |  |  |  |  |  |  |  |
| Potassium Hydroxide |  | 1310-58-3 | 1 | 0 | D002 | 0 | 0.5 | 2.5 |
| # DPD Free Chlorine Reagent | chlorine indicator |  |  |  |  |  |  |  |
| Salt of N,N-Diethyl-p-Phenylenediamine |  |  | * | * | * | * | * | * |
| Carboxylate Salt |  |  | * | * | * | * | * | * |
| Sodium Phosphate, Dibasic |  |  | DUST | 0 | 0 | 0 | 0 | 1 |
| Ethylenediaminetetraacetic Acid, Disodium Salt |  |  | DUST | 0 | 0 | 0 | 0 | 1 |
| Electric Motor Cleaner (newer) | Lectra-Clean |  |  |  |  |  |  |  |
| Paraffinic hydrocarbon |  |  | 1 | 0 | D001 | 0 | 0 | 2 |
| Propylene glycol |  | 57-55-6 | 0 | 0 | 0 | 0 | 0 | 0 |
| Electric Motor Cleaner (old) | Lectra-Clean |  |  |  |  |  |  |  |
| 1,1,1 Trichlorethane |  | 71-55-6 | 1 | 1 | F001 | 0 | 0 | 3 |
| Perchloroethylene |  | 127-18-4 | 1 | 0 | F002 | 0 | 0 | 2 |
| # Food Machinery Lubricant |  |  |  |  |  |  |  | 0 |
| Gallic Acid |  | 149-91-7 | 0 | 0 | 0 | 0 | 0 | 0 |
| Gas Leak Detector |  |  | 1 | 0 | D001 | 0 | 0 | 2 |
| Isopropanol |  | 67-63-0 | 1 | 0 | D001 | 0 | 0 | 2 |
| # Dichlorodifluoromethane | Refrigerant | 75-71-8 | 1 | 0 | U075 F002 | 0 | 1 | 3 |
| # Trichloromonofluoromethane | Refrigerant | 75-69-4 | 1 | 0 | F002 | 0 | 1 | 3 |
| # Chorotrifluoromethane | Refrigerant | 75-72-9 | 0 | 0 | F002 | 0 | 1 | 2 |
| # Grease Cartridge |  |  |  |  |  |  |  |  |
| Petroleum oil |  |  | 0 | 0 | 0 | 0 | 0 | 0 |
| # Hardi-Cube Salt | De-Icer |  |  |  |  |  |  |  |
| Sodium Chloride |  | 7647-14-5 | 0 | 0 | 0 | 0 | 1 | 1 |
| Sodium Ferrocyanide |  | 13601-19-9 | 0 | 0 | 0 | 0 | 1 | 1 |
| Sodium Carbonate |  | 497-19-8 | DUST | 0 | 0 | 0 | 1 | 2 |
| # Hardness Indicator Buffer |  |  |  |  |  |  |  |  |
| Triethanolamine |  | 102-71-6 | 0 | 0 | 0 | 0 | 0 | 0 |
| Ammonium Hydroxide | ammonia | 1336-21-6 | 1 | 0 | D002 | 0 | 0 | 2 |
| # Humidifier Water Treatment | Humiditreat |  |  |  |  |  |  |  |
| # Hydrated Lime | calcium hydroxide | 1305-62-0 | 1 | 0 | 0 | 0 | 1 | 2 |
| # Industrial Cleaner, pH 12-12-5 |  |  |  |  |  |  |  |  |
| Sodium Xylene Sulfonate |  | 1300-72-7 | * | * | * | * | * | * |
| Tetrasodium Salt of EDTA |  | 64-02-8 | 0 | 0 | 0 | 0 | 1 | 1 |
| Alkoxylated Linear Alcohol |  | 68439-50-9 | * | * | * | * | * | * |
| Monoethanolamine |  | 141-43-5 | 1 | 0 | D002 | 0 | 1 | 3 |
| 2-Butoxyethanol |  | 111-76-2 | 1 | 0 | D002 | 0 | 1 | 3 |
| # Inhibitor | Betz Dearborn |  |  |  |  |  |  |  |
| Petroleum distillates |  | 6474-52-5 | 1 | 0 | D001 | 0 | 0 | 2 |
| Sodium petroleum sulfonate |  | 68608-26-4 | * | * | * | * | * | * |
| Iodide-Iodate |  |  |  |  |  |  |  |  |
| # Isopropyl Alcohol | isopropanol | 67-63-0 | 1 | 0 | D001 | 0 |  | 2 |
| # Kerosene |  | 8008-20-6 | 0 | 0 | D001 | 0 | 0 | 1 |
| # Lacquer | aerosol Touch-up Paint |  |  |  |  |  |  |  |
| Acetone |  | 67-64-1 | 1 | 0 | D001 | 0 | 0.5 | 2.5 |
| Butyl Acetate |  | 123-86-4 | 1 | 0 | D001 | 0 | 0.5 | 2.5 |
| Methyl Ethyl Ketone |  | 78-93-3 | 1 | 0 | D001 | 0 | 0.5 | 2.5 |
| Propane |  | 74-98-6 | 1 | 0 | D001 | 0 | 0.5 | 2.5 |
| Toluene |  | 108-88-3 | 1 | 0 | D001 | 0 | 0.5 | 2.5 |
| Xylene |  | 1330-20-7 | 1 | 0 | D001 | 0 | 0.5 | 2.5 |
| Ethyl 3-ethoxypropionate |  | 763-69-9 | 0 | 0 | D001 | 0 | 0.5 | 1.5 |
| Ethyl Acetate |  | 141-78-6 | 1 | 0 | D018 | 0 | 0.5 | 2.5 |
| Ethanol |  | 74-17-5 | 1 | 0 | D001 | 0 | 0.5 | 2.5 |
| Isopropyl Alcohol |  | 67-63-0 | 1 | 0 | D001 | 0 | 0.5 | 2.5 |
| Methyl Amyl Ketone |  | 110-43-0 | 1 | 0 | D001 | 0 | 0.5 | 2.5 |
| Ethylbenzene |  | 100-41-4 | 1 | 0 | D001 | 0 | 0.5 | 2.5 |
| Lampblack |  | 1333-86-4 | 1 | 0 | 0 | 0 | 0 | 1 |
| Iron Oxide |  | 51274-00-1 | 1 | 0 | D001 | 0 | 0 | 2 |
| Titanium Dioxide |  | 13463-67-7 | 1 | 0 | 0 | 0 | 0 | 1 |
| # Lacquer Thinner |  |  |  |  |  |  |  |  |
| Toluene |  |  | 1 | 0 | F005 | 0 | 0 | 2 |
| Acetone |  |  | 1 | 0 | F003 | 0 | 0 | 2 |
| # Lead Acid Battery |  |  |  |  |  |  |  |  |
| Lead |  |  | 1 | 1 | D008 | 0 | 0.5 | 3.5 |
| Sulfuric Acid |  |  | 1 | 0 | D002 | 0 | 0.5 | 2.5 |
| # Low Hardness Buffer |  |  |  |  |  |  |  |  |
| Ethanolamine |  | 141-43-5 | 1 | 0 | D002 | 0 | 0 | 2 |
| Hydrogen Chloride |  | 7697-01-0 | 1 | 0 | D003 | 0 | 0 | 2 |
| # Low Hardness Indicating Powder |  |  | 0 | 0 | 0 | 0 | 0 | 0 |
| # Low Hardness Titrating Solution |  |  | 0 | 0 | 0 | 0 | 0 | 0 |
| # Lubricating Oils |  |  |  |  |  |  |  |  |
| Petroleum or Hydrocarbon Oil |  |  | 1 MIST | 0 | 0 | 0 | 0.5 | 1.5 |
| # Mapp Gas | Bernz Omatic |  |  |  |  |  |  |  |
| # Methylene Chloride | paint stripper, Zip Strip | 75-09-2 | 1 | 0 | F002 | 0 | 0 | 2 |
| # Methyl Red Indicator |  |  |  |  |  |  |  |  |
| Isopropyl Alcohol (Isopropanol) |  | 67-63-0 | 1 | 0 | D001 | 0 | 0 | 2 |
| # Microbiocide |  |  |  |  |  |  |  |  |
| 1-bromo-3-chloro-5-dimethylhydantoin |  |  | 0 | 0 | PEST | 0 | 1 | 2 |
| # Mineral Spirits | parts cleaner paint thinner |  | 1 | 0 | D001 | 0 | 1 | 3 |
| # Muriatic Acid | Kleen Strip, cement cleaner |  | 1 | 0 | D002 | 0 | 1 | 3 |
| # Neutralizing Amine | boiler chemical |  |  |  |  |  |  |  |
| Diethylaminoethanol |  | 100-37-8 | 1 | 0 | D001 | 0 | 1 | 3 |
| Cyclohexylamine |  | 108-91-8 | 1 | 0 | D001 | 0 | 1 | 3 |
| Nitric Acid | nitric acid | 7697-37-2 | 1 | 0 | D002 | 0 | 0 | 2 |
| # Nitrous Oxide |  | 10024-97-2 | 0 | 0 | D001 | 0 | 0 | 1 |
| # Penetrating Oil |  |  |  |  |  |  |  |  |
| # Oxygen Gas | Oxygen |  | 0 | 0 | D001 | 0 | 1 | 2 |
| # Oxygen Scavenger | boiler chemical |  |  |  |  |  |  |  |
| Sodium Sulfite |  | 7757-83-7 | 1 | 0 | 0 | 0 | 0.5 | 1.5 |
| # Paint | Yellow Traffic Marking Paint |  |  |  |  |  |  |  |
| Calcium Carbonate |  | 1317-65-3 | 1 | 0 | 0 | 0 | 0.5 | 1.5 |
| WM&P Naphtha Solvent Naphtha |  | 64742-47-8 | 1 | 0 | D001 | 0 | 0.5 | 2.5 |
| Hydrous Magnesium Silicate |  | 14807-96-6 | 1 | 0 | 0 | 0 | 0.5 | 1.5 |
| Lead Chromate | Ci Pigment Yellow 34 | 7758-97-6 | 1 | 1 | D008 D007 | 0 | 1 | 4 |
| # Penetrating Oil | Sprasolvo |  | 0 | 0 | 0 | 0 | 0 | 0 |
| # Phenolphthalein Indicator |  |  | 0 | 0 | 0 | 0 | 0 | 0 |
| Isopropyl Alcohol |  |  | 1 | 0 | D001 | 0 | 0 | 2 |
| # Phosphate Donor | cooling water treatment |  | 0 | 0 | 0 | 0 | 0 | 0 |
| EDTA (Ethylenediaminetetraacetic acid) |  |  | 0 | 0 | 0 | 0 | 0 | 0 |
| Phosphate Donor |  |  | 0 | 0 | 0 | 0 | 0 | 0 |
| Phosphate Reagent |  |  | 0 | 0 | 0 | 0 | 0 | 0 |
| # Sulfuric Acid |  |  | 1 | 0 | D002 | 0 | 0 | 2 |
| Phosphates EDTA phosphate | MCP, boiler chemical |  | 0 | 0 | 0 | 0 | 1 | 1 |
| # Plastic Adhesive |  |  |  |  |  |  |  |  |
| Methyl Ethyl Ketone |  | 78-93-3 | 1 | 0 | F003 | 0 | 1 | 3 |
| Vinyl Chloride-Vinyl Acetate Polymer |  | 9003-22-9 | 0 | 0 | 0 | 0 | 1 | 1 |
| Polyurethane Polymer |  | 56815-45-3 | * | * | * | * | 1 | *1 |
| Potassium Chromate Indicator (chromium compounds) |  |  | 1 | 1 | D007 | 0 | 0 | 3 |
| # Potassium Hydroxide Solution | drain cleaner |  |  |  |  |  |  |  |
| Potassium Hydroxide |  | 1310-58-3 | 1 | 0 | D002 | 0 | 0 | 2 |
| Propane Fuel | Propane | 74-98-6 | 1 | 0 | D001 | 0 | 1 | 3 |
| Propane, Instrument Grade |  | 74-98-6 | 1 | 0 | D001 | 0 | 1 | 3 |
| # R-9C | sodium sulfite | 7757-83-7 | 1 | 0 | D001 | 0 | 0.5 | 2.5 |
| # R-10 | sodium sulfite | 7757-83-7 | 1 | 0 | D001 | 0 | 1 | 3 |
| # R-10-C | sodium sulfite | 7757-83-7 | 1 | 0 | D001 | 0 | 1 | 3 |
| # Bromotrifluoromethane | Refrigerant | 75-63-8 | 1 | 0 | D001 | 0 | 1 | 3 |
| # R-503 |  |  |  |  |  |  |  | 0 |
| Trifluormethane Chlorotrifluromethane | Refrigerant | 75-46-7/75-72-0 | 1 | 0 | F002 | 0 | 1 | 3 |
| # Refrigerant Oil |  |  |  |  |  |  | 0 | 0 |
| # Rust Inhibitor | Rid Rust |  |  |  |  |  | 0 | 0 |
| Petroleum Distillates |  |  | 1 | 0 | D001 | 0 | 0 | 2 |
| # Scale Cleaner | boiler chemical, Kleen |  |  |  |  |  |  |  |
| Sulfuric Acid |  | 5329-14-6 | 1 | 0 | D002 | 0 | 1 | 3 |
| Ethylene Glycol |  | 107-21-1 | 1 | 0 | 0 | 0 | 1 | 2 |
| Amorphous Silica |  | 112945-52-5 | 1 | 0 | 0 | 0 | 1 | 2 |
| # Semigloss Paint - Latex |  |  |  |  |  |  |  |  |
| Titanium Dioxide |  | 13463-67-7 | NA | 0 | 0 | 0 | 0.5 | 0.5 |
| Hydrous Aluminum Silicate |  | 66402-68-4 | NA | 0 | 0 | 0 | 0.5 | 0.5 |
| # Silicone Lubricant |  |  | 0 | 0 | 0 | 0 | 0 | 0 |
| # Silicone Sealant |  |  |  |  |  |  |  |  |
| Dimethylformamide |  | 00068-12-2 | 1 | 0 | D001 | 0 | 0.5 | 2.5 |
| Acetamido-silane |  | NE | * | 0 | * | * | 0.5 | *0.5 |
| Calcium Carbonate |  | 00471-34-1 | NA | 0 | 0 | 0 | 0.5 | 0.5 |
| Polydimethylsiloxane, Hydroxy-Terminated |  | 70131-67-8 | * | 0 | * | * | 0.5 | *0.5 |
| Methylvinyl bis-silane |  | 50791-87-2 | * | 0 | * | * | 0.5 | *0.5 |
| Dimethyl methyldiethyl N-hydroxyethamnine Silicone Polymer |  | 68952-53-4 | * | 0 | * | * | 0.5 | *0.5 |
| # Sodium Hydroxide Solution | SW-21L, boiler chemical |  |  |  |  |  |  |  |
| Sodium Hydroxide |  | 1310-73-2 | 1 | 0 | D002 | 0 | 1 | 3 |
| Borates, Tetra, Sodium Salt, Pentahydrate |  | 1303-96-4 | 1 | 0 | D001 | 0 | 1 | 3 |
| Sodium Nitrite |  | 7632-00-0 | 0 | 0 | 0 | 0 | 1 | 1 |
| 2-Mercaptobenzothiazole |  | 149-30-4 | DUST | 0 | PEST | 0 | 1 | 3 |
| Sodium Thiosulfate |  | 7772-98-7 | 0 | 0 | 0 | 0 | 0 | 0 |
| # Solvent | parts cleaner, paint thinner |  |  |  |  |  |  |  |
| Toluene |  | 108-88-3 | 1 | 0 | F005 | 0 | 1 | 3 |
| Spray Adhesive |  |  |  |  |  |  |  | 0 |
| Nitrocellulose Resin |  |  | 0 | 0 | 0 | 0 | 0 | 0 |
| Trichloroethane |  |  | 1 | 1 | F002 | 0 | 0 | 3 |
| # Standard Potassium Permanganate |  |  | 0 | 0 | D001 | 0 | 0 | 1 |
| # Standard Silver Nitrate |  | 7761-88-8 | 0 | 0 | D001 | 0 | 0 | 1 |
| # Standard Thorium Nitrate** | fluorine determination. | 13823-29-5 | **0 | 0 | ** | 0 | 0 | **0 |
| # Starch Indicator Acid Powder |  |  | 1 | 0 | 0 | 0 | 0 | 1 |
| # Stoddard Solvent | Safety-Kleen Premium Solvent |  |  |  |  |  |  |  |
| Petroleum Distillates |  | 64742-47-8 | 1 | 0 | D001 | 0 | 1 | 3 |
| # Sulfuric Acid | sulfuric Acid | 7664-93-9 | 1 | 0 | D002 | 0 | 0 | 2 |
| # Thinner Paint | Thinzit |  |  |  |  |  |  | 0 |
| Toluene |  |  | 1 | 0 | F005 | 0 | 0.5 | 2.5 |
| Acetone |  |  | 1 | 0 | F003 | 0 | 0.5 | 2.5 |
| # Thymol Blue |  |  | 0 | 0 | 0 | 0 | 0 | 0 |
| # Trisodium phosphate | TSP Cleaner | 7601-54-9 | 0 | 0 | 0 | 0 | 1 | 1 |
| # Turbine Oil | PQ-68 |  | 0 | 0 | 0 | 0 | 1 | 1 |
| # Tuner Degreaser | Tun-O-Wash |  |  |  |  |  |  | 0 |
| Hexane |  | 110-54-3 | 1 | 0 | D001 | 0 |  | 2 |
| Isohexane |  | 107-83-5 | 0 | 0 | D001 | 0 |  | 1 |
| 3-methyl pentane |  | 96-14-0 | 0 | 0 | D001 | 0 |  | 1 |
| 2,3- dimethylbutane |  | 79-29-8 | 0 | 0 | D001 | 0 |  | 1 |
| Neohexane |  | 75-83-2 | 0 | 0 | D001 | 0 |  | 1 |
| Ethyl acetate |  | 141-78-6 | 1 | 0 | D001 | 0 |  | 2 |
| Isopropyl Alcohol |  | 67-63-0 | 1 | 0 | D001 | 0 |  | 2 |
| Methyl alcohol |  | 67-56-1 | 1 | 0 | D001 | 0 |  | 2 |
| Ethyl alcohol |  | 64-17-5 | 1 | 0 | D001 | 0 |  | 2 |
| # Urethane Enamel | oil-based paint |  |  |  |  |  |  |  |
| Mineral Spirits |  | 8052-41-3 | 1 | 0 | D001 | 0 | 0.5 | 2.5 |
| Solvent Naphtha |  | 64742-95-6 | 1 | 0 | D001 | 0 | 0.5 | 2.5 |
| Xylene |  | 1330-20-7 | 1 | 0 | F001 | 0 | 0.5 | 1.5 |
| # Xylenol Orange |  |  |  |  |  |  |  | 0 |
| Xylene |  | 1330-20-7 | 1 | 0 | F003 D001 | 0 | 0.5 | 2.5 |

LABORATORY Prioritization Tool

| CHEMICAL NAME & INGREDIENTS | COMMON/TRADE NAME OR USE | ### CAS# | OSHA | PBT | RCRA | P? | VOL | TOTAL |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Acetic acid |  | 64197 | 1 | 0 | D002 | 0 | 0 | 2 |
| Acetic anhydride |  | 108247 | 1 | 0 | D001 | 0 | 0 | 2 |
| Acetone |  | 67641 | 1 | 0 | F003, U002 | 0 | 1 | 3 |
| Acid fuchsin, aqueous |  |  | 0 | 0 | 0 | 0 | 0 | 0 |
| Acrylamide |  | 79061 | 1 | 0 | D001 | 0 | 0 | 2 |
| Alcian blue* |  |  |  |  |  |  | 0 | * |
| Alcian yellow* |  |  |  |  |  |  | 0 | * |
| Alcohol, ethanol |  |  | 1 | 0 | D001 | 0 | 1 | 3 |
| Alcohol, methanol |  |  | 1 | 0 | F004 D001 U154 | 0 | 0.5 | 2.5 |
| Aluminum ammonium sulfate |  | 7429905 | 1 | 0 | D001 | 0 | 0 | 2 |
| Americlear |  |  | 1 | 0 | D001 | 0 | 1 | 3 |
| Ammonium hydroxide |  | 1336216 | 1 | 0 | D002 | 0 | 0.5 | 2.5 |
| Ammonium lauryl sulfate |  | 2235543 | 0 | 0 | 0 | 0 | 0 | 0 |
| Ammonium oxylate |  |  | 0 | 0 | 0 | 0 | 0 | 0 |
| Aniline blue solution, aqueous* |  |  |  |  |  |  | 0 | * |
| Arsenic |  | 7440382 | 1 | 1 | D004 | 0 | 0 | 3 |
| Barbital |  | 57443 | 0 | 0 | 0 | 0 | 0 | 0 |
| Barbituric acid, 5,5-Diethyl-, Sodium Salt |  | 67527 | 0 | 0 | 0 | 0 | 0 | 0 |
| Barium sulfate |  | 7727437 | 1 | 0 | D005 | 0 | 0 | 1 |
| Benzene |  | 71432 | 1 | 0 | U019, D018 | 0 | 0 | 2 |
| Benzethonium chloride |  | 121540 | 0 | 0 | PEST | 0 | 0 | 1 |
| Benzidine |  | 92875 | 1 | 0 | U002 | 0 | 0 | 2 |
| Benzyl alcohol |  | 100516 | 0 | 0 | 0 | 0 | 0 | 0 |
| Biebrich scarlet* |  |  |  |  |  |  |  | * |
| Biphenylol, sodium salt, 2- |  | 92886 | 0 | 0 | 0 | 0 | 0 | 0 |
| Borax solution |  |  | 0 | 0 | 0 | 0 | 0 | 0 |
| Butanol |  | 71363 | 1 | 0 | U131 | D001 | 0 | 2 |
| Butyl acetate |  | 123864 | 1 | 0 | D001 | 0 | 0 | 2 |
| Butyl alcohol, sec- |  | 78922 | 1 | 0 | D001 | 0 | 0 | 2 |
| Butylamine |  | 109739 | 1 | 0 | D001 | 0 | 0 | 2 |
| Caffeine |  | 58082 | 0 | 0 | 0 | 0 | 0 | 0 |
| Calcium carbonate |  | 471341 | 1 | 0 | 0 | 0 | 0 | 1 |
| Calcium sulfate |  | 7778189 | 1 | 0 | 0 | 0 | 0 | 1 |
| Carbon tetrachloride |  | 56235 | 1 | 1 | U211, F002 | 0 | 0 | 3 |
| Cetylpyridinium chloride |  | 6004246 | 0 | 0 | 0 | 0 | 0 | 0 |
| Chloroform |  | 67663 | 1 | 1 | D022 U044 | 0 | 0.5 | 3.5 |
| Chromic acid |  | 1333820 | 1 | 1 | D007 | 0 | 0.5 | 3.5 |
| Chromium trioxide |  | 1333820 | 1 | 1 | D007 | 0 | 0 | 3 |
| Citric acid |  | 77929 | 0 | 0 | 0 | 0 | 0 | 0 |
| Cobaltous acetate |  | 71487 | 0 | 0 | 0 | 0 | 0 | 0 |
| Congo red |  |  | 0 | 0 | 0 | 0 | 0 | 0 |
| Copper (II) sulfate (1:1) |  | 7758987 | 0 | 1 | 0 | 0 | 0 | 1 |
| Crystal violet* |  |  |  |  |  |  |  | * |
| Cyclohexanone |  | 108941 | 1 | 0 | D001 | 0 | 0 | 2 |
| Di (ethylhexyl) phthalate |  | 117-81-7 | 1 | 1 | U028 | 0 | 1 | 4 |
| 1,2 dichloroethane |  | 107062 | 1 | 0 | F002 | 0 | 0 | 2 |
| Diethylamine |  | 109897 | 1 | 0 | D001 | 0 | 0 | 2 |
| Dioxane, 1,4- |  | 123911 | 1 | 0 | U109 | 0 | 0 | 2 |
| Eosin Y |  | 548-26-5 | 0 | 0 | 0 | 0 | 0 | 0 |
| Ethyl acetate |  | 141786 | 1 | 0 | D001 | 0 | 0 | 2 |
| Ethyl alcohol |  | 64175 | 1 | 0 | D001 | 0 | 1 | 3 |
| Ethylene glycol |  | 10020436 | 1 | 0 | 0 | 0 | 0 | 1 |
| Ethylene oxide |  | 75-21-8 | 1 | 0 | U115 D001 | 0 | 0 | 2 |
| Ethylenediaminetetraacetic acid, |  | 67401507 | 0 | 0 | 0 | 0 | 1 | 1 |
| Ferric chloride |  | 7705-08-0 | 1 | 0 | D002 | 0 | 0 | 2 |
| Formaldehyde |  | 50-00-0 | 1 | 0 | U122 D001 | 0 | 1 | 3 |
| Formamide, N,N-dimethyl- |  | 75-12-7 | 1 | 0 | D001 | 0 | 0 | 2 |
| Formic acid |  | 64-18-6 | 1 | 0 | D002 | 0 | 0 | 2 |
| fuchsin |  | 569-61-9 | 0 | 0 | 0 | 0 | 0 | 0 |
| Giemsa stain |  |  | 0 | 0 | 0 | 0 | 0 | 0 |
| Glutaraldehyde |  | 111-30-8 | 1 | 0 | PEST | 0 | 0 | 2 |
| Gold chloride, aqueous |  |  | 0 | 0 | 0 | 0 | 0 | 0 |
| Gum mastic |  |  | 0 | 0 | D001 | 0 | 0 | 1 |
| Hematoxylin, Harris |  |  | 0 | 1** | 1 | 0 | 0.5 | 2.5 |
| Hydrazine sulfate |  | 10034-93-2 | 0 | 0 | 0 | 0 | 0 | 0 |
| Hydrochloric acid |  | 7647010 | 1 | 0 | D002 | 0 | 0.5 | 2.5 |
| Hydrogen peroxide |  |  | 1 | 0 | D001 | 0 | 0 | 1 |
| Iodine |  | 7553-56-2 | 1 | 0 | 0 | 0 | 0 | 2 |
| Isopropyl alcohol |  | 67630 | 1 | 0 | D001 | 0 | 0.5 | 2.5 |
| Isopropylamine |  | 75310 | 1 | 0 | D001 | 0 | 0 | 2 |
| Lactic acid |  | 50215 | 0 | 0 | 0 | 0 | 0 | 0 |
| Lactose |  | 63423 | 0 | 0 | 0 | 0 | 0 | 0 |
| Lead acetate (lead compounds) |  | 15347-57-6 | 0 | 1 | D008 U144 | 0 | 0 | 2 |
| Leucine |  | 61-90-5 | 0 | 0 | 0 | 0 | 0 | 0 |
| Light green stain, aqueous |  |  | 0 | 0 | 0 | 0 | 0 | * |
| Lithium carbonate |  | 554-13-2 | DUST | 0 | D003 | 1 | 0 | 2 |
| Lithium chloride |  | 7447-41-8 | DUST | 0 | D003 | 0 | 0.5 | 2.5 |
| Luxol fast blue |  |  |  |  |  |  | 0 | * |
| Maleic anhydride |  | 108-31-6 | 1 | 0 | D001 | 0 | 0 | 2 |
| Manganese chloride |  | ###### | 1 | 0 | 0 | 0 | 0 | 1 |
| Mercuric chloride |  | 7487947 | 1 | 1 |  | 0 | 0 | 3 |
| Mercuric nitrate |  | 7783348 | 1 | 1 | D009 | 0 | 0 | 3 |
| Mercuric oxide |  | 21908532 | 1 | 1 | D009 | 0 | 0.5 | 3.5 |
| Mercury, ((o-carboxyphenyl)thio)ethyl-, |  | 7439976 | 1 | 1 | D009 | 0 | 0 | 3 |
| Methenamine |  | 100-97-0 | 0 | 0 | D009 | 0 |  | 1 |
| Methyl alcohol |  | 67561 | 1 | 0 | F004 D001 | 0 | 1 | 3 |
| Methyl paraben |  | 99-76-3 | 0 | 0 | 0 | 0 | 0 | 0 |
| Methylene blue |  | 61-73-4 | 0 | 0 | 0 | 0 | 0 | 0 |
| Molybdenum trioxide |  | 1313-27-5 | 1 | 0 | 0 | 0 | 0 | 1 |
| Morpholine |  | 110-91-8 | 1 | 0 | D001 | 0 | 0 | 2 |
| Naphthylamine, alpha |  | 134-32-7 | 1 | 0 | D001 | 0 | 0 | 2 |
| Napthol, alpha- |  | 90-15-3 | 0 | 0 | 0 | 0 | 0 | 0 |
| n-Butyl alcohol |  | 71-36-3 | 1 | 0 | D001 | 0 | 0 | 2 |
| Nitric acid |  | 7697-37-2 | 1 | 0 | D002 | 0 | 0 | 2 |
| Nitrilotriethanol, 2,2',2''- |  | 102-71-6 | 0 | 0 | 0 | 0 | 0 | 0 |
| n-Propyl alcohol |  | 71-23-8 | 1 | 0 | D001 | 0 | 0 | 2 |
| Oil red O |  | 3118-97-6 | 0 | 0 | 0 | 0 | 0 | 0 |
| Oxalic acid |  | 144-62-7 | 1 | 0 | D002 | 0 | 0 | 2 |
| Pentyl alcohol |  | 75-85-4 |  | 0 | D001 | 0 | 0 | 1 |
| Phenylhydrazine hydrochloride |  | 59-88-1 | 0 | 0 | 0 | 0 | 0 | 1 |
| Phenylmercuric acetate |  | 62384 | 1 | 1 | P092 | 1 | 0 | 4 |
| Phenol |  | 108952 | 1 | 1 | U188 PEST | 1 | 0 | 4 |
| Phosphoric acid |  | 7664382 | 1 | 0 | D002 | 0 | 0 | 2 |
| Phosphotungstic acid |  | 12067-99-1 | 0 | 0 | 0 | 0 | 0 | 0 |
| Picric acid |  | 88891 | 10 | 0 | D001 | 0 | 0.5 | 2.5 |
| Piperidine |  | 110894 | 0 | 0 | D001 | 0 | 0 | 1 |
| Potassium cyanide |  | 151508 | 1 | 1 | P099 | 1 | 0 | 4 |
| Potassium ferrocyanide |  |  | 1 | 1 | D003 | 0 | 0 | 3 |
| Potassium hydroxide |  | 1310-58-3 | 1 | 0 | D002 | 0 | 0 | 2 |
| Potassium iodate |  | ###### | 0 | 0 | 0 | 0 | 0 | 0 |
| Potassium permanganate |  | 7722-64-7 | 1 | 0 | D001 | 0 | 0 | 1 |
| Potassium thiocyanate |  | 333-20-0 | 1 | 0 | 0 | 0 | 0 | 1 |
| Propane |  | 74-98-6 | 1 | 0 | D001 | 0 | 0 | 2 |
| Propionic acid |  | 79-09-4 | 0 | 0 | 0 | 0 | 0 | 0 |
| Propylene oxide |  | 75-56-9 | 1 | 0 | D001 | 0 | 0 | 2 |
| Pyridine |  | 110-86-1 | 1 | 0 | U196 | 0 | 0 | 2 |
| Pyrogallic acid |  | 87-66-1 | 0 | 0 | 0 | 0 | 0 | 0 |
| Rhodamine | Acid Red | 37299-86-8 | 0 | 0 | 0 | 0 | 0 | 0 |
| Safranin, aqueous |  |  | 0 | 0 | D011 | 0 | 0 | 1 |
| Selenium |  |  | 0 | 1 | D017 | 0 | 0 | 2 |
| Silica |  | 11292-60-08 | 1 | 0 | 0 | 0 | 0 | 1 |
| Silver nitrate |  | 776-18-88 | 0 | 0 | D011 | 0 | 0 | 1 |
| Sodium acetate |  |  | DUST | 0 | 0 | 0 | 0 | 1 |
| Sodium azide |  | 26628228 | 1 | 0 | P015 | 1 | 1 | 4 |
| Sodium benzoate |  | 532321 | 0 | 0 | 0 | 0 | 0 | 0 |
| Sodium bisulfite |  |  | 0 | 0 | 0 | 0 | 0 | 0 |
| Sodium chloride |  |  | 0 | 0 | 0 | 0 | 0 | 0 |
| Sodium cyanide |  | 143339 | 1 | 1 | P016, D003 | 1 | 0 | 4 |
| Sodium dodecylbenezenesulfonate |  | 2515300 | 0 | 0 | 0 | 0 | 0 | 0 |
| Sodium fluoride |  | 7681494 | 1 | 0 | D001 | 0 | 0.5 | 2.5 |
| Sodium hydroxide |  | 1310732 | 1 | 0 | D002 | 0 | 0 | 2 |
| Sodium hypochlorite | Bleach | 7681529 | 0 | 0 | 0 | 0 | 1 | 1 |
| Sodium metabisulfite |  |  | 0 | 0 | 0 | 0 | 0 | 0 |
| Sodium nitrate |  | 7631994 | DUST | 0 | D001 | 0 | 0 | 2 |
| Sodium nitrite |  | 7632000 | DUST | 0 | D001 | 0 | 0 | 2 |
| Sodium phosphate diabasic |  |  | DUST | 0 | 0 | 0 | 0 | 0 |
| Sodium phosphate monobasic |  |  | DUST | 0 | 0 | 0 | 0 | 0 |
| Sodium thiosulfate |  |  | 0 | 0 | 0 | 0 | 0 | 0 |
| Sorbic acid |  | 110441 | 0 | 0 | 0 | 0 | 0 | 0 |
| Succinic acid |  | 110156 | 0 | 0 | 0 | 0 | 0 | 0 |
| Sudan black B |  |  | 0 | 0 | 0 | 0 | 0 | 0 |
| Sudan IV |  |  | 0 | 0 | 0 | 0 | 0 | 0 |
| Sulfanilamide |  | 63741 | 0 | 0 | 0 | 0 | 0 | 0 |
| Sulfur dioxide |  | 7446095 | 1 | 0 | 1 | 0 | 0 | 2 |
| Sulfuric acid |  | 7664939 | 1 | 0 | D002 | 0 | 0.5 | 2.5 |
| Thimerosal (as reagent preservative)** |  |  | 0 | 1** | D009 | 0 | 1 | 3 |
| Thioacetamide |  | 62555 | 0 | 0 | U218 | 0 | 0 | 0 |
| Thiosemicarbazide |  | 79196 | 0 | 0 | P116 | 1 | 0 | 2 |
| Thiourea |  | 62566 | 0 | 0 | U219 | 0 | 0.5 | 1.5 |
| Toluene |  | 108883 | 1 | 0 | U020 F005 | 0 | 0.5 | 2.5 |
| Toluidine blue solution* |  |  |  |  |  |  |  | * |
| Toluidine, ortho- |  | 95534 | 1 | 0 | U328 | 0 | 0 | 2 |
| Trichloroacetic acid |  | 76039 |  | 0 | D002 | 0 | 0 | 1 |
| Trichloroethane, 1,1,1- |  | 79005 | 1 | 1 | F001 | 0 | 0 | 3 |
| Trichloroethylene |  | 79016 | 1 | 0 | U228 F002 | 0 | 0 | 2 |
| Xylene |  | 108383 | 1 | 0 | F003 U239 | 0 | 1 | 3 |
| Zinc acetate |  | 557346 | 0 | 1 | 0 | 0 | 0 | 1 |
| Zinc sulfate (1:1) |  | 7446197 | 0 | 1 | D001 | 0 | 0 | 2 |

NUTRITION SERVICES Prioritization Tool

| PRODUCT TYPE/HAZARDOUS INGREDIENTS | COMMON/TRADE NAME OR USE | CAS# | OSHA | PBT | RCRA | P? | VOL | TOTAL |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # Bleach |  |  |  |  |  |  |  |  |
| Sodium Hypochlorite |  | 7681-92-9 | 1 | 0 | D003 | 0 | 1 | 1 |
| Sodium Hydroxide |  | 1310-73-2 | 1 | 0 | D002 | 0 | 1 | 3 |
| ### Break-Up |  |  |  |  |  |  |  |  |
| Potassium Hydroxide |  | 1310-58-3 | 1 | 0 | D002 | 0 | 1 | 3 |
| Trisodium Salt of NTA |  |  | 0 | 0 | 0 | 0 | 1 | 1 |
| Metasilicate |  | 6834-92-0 | 1 | 0 | D002 | 0 | 1 | 3 |
| Ethylene Glycol Monobutyl Ether |  | 111-76-2 | 1 | 0 | 0 | 0 | 1 | 2 |
| # Cleaner and degreaser, multipurpose |  |  |  |  |  |  |  |  |
| Ethanolamine (MEA) |  | 141-43-5 | 1 | 0 | D002 | 0 | 0 | 2 |
| Diethylene Glycol Ethyl Ether |  | 111-90-0 | 0 | 0 | 0 | 0 |  |  |
| Chelating Agents |  |  | * | * | * | * |  |  |
| Trisodium Nitrilotriacetate |  | 5064-31-3 | 0 | 0 | 0 | 0 |  |  |
| ### Cleaner, multipurpose |  |  |  |  |  |  |  |  |
| Dipropylene Glycol Methyl Ether |  | 34590-94-8 | 1 | 0 | 0 | 0 | 1 | 2 |
| Sodium Metasilicate |  | 6834-92-0 | 1 | 0 | D003 | 0 | 1 | 3 |
| Sodium tripolyphosphate |  | 7758-29-4 | 0 | 0 | 0 | 0 | 1 | 1 |
| ### Cleaner, multipurpose liquid |  |  |  |  |  |  |  |  |
| Nonylphenol ethoxylate |  | 9016-45-9 | 0 | 0 | 0 | 0 | 1 | 1 |
| # Cold Temperature Cleaner |  |  |  |  |  |  |  |  |
| Ethanolamine (MEA) |  | 141-43-5 | 1 | 0 | D002 | 0 | 1 | 3 |
| Isopropyl Alcohol |  | 67-63-0 | 1 | 0 | D001 | 0 | 1 | 3 |
| ### Delimer/descaler, acidic |  |  |  |  |  |  |  |  |
| Phosphoric acid |  | 7664-38-2 | 1 | 0 | D002 | 0 | 1 | 3 |
| ### Delimer/descaler, acidic |  |  |  |  |  |  |  |  |
| Hydrogen Chloride |  | 7647-01-0 | 1 | 0 | D002 | 0 | 1 | 3 |
| Phosphoric acid |  | 7664-38-2 | 1 | 0 | D002 | 0 | 1 | 3 |
| ### Detergent, acid |  |  |  |  |  |  |  |  |
| Phosphoric Acid |  | 7664-38-21 | 1 | 0 | D002 | 0 | 1 | 3 |
| Nitric Acid |  | 7697-37-21 | 1 | 0 | D002 | 0 | 1 | 3 |
| Nickel sulfate hexahydrate |  | 7786-81-41 | 1 | 1 | D002 | 0 | 1 | 4 |
| ### Detergent, automatic, chlorinated |  |  |  |  |  |  |  |  |
| Sodium Hydroxide Pellets |  | 1310-73-2 | 1 | 0 | D002 | 0 | 1 | 3 |
| Sodium dichloro-s-triazinetrione dihydrate |  | 51580-86-0 | 0 | 0 | PEST | 0 | 1 | 2 |
| Sodium Metasilicate |  | 6834-92-0 | 1 | 0 | D002 | 0 | 1 | 3 |
| Sodium Carbonate |  | 497-19-8 | DUST | 0 | 0 | 0 | 1 | 2 |
| ### Detergent, automatic, dishmachine |  |  |  |  |  |  |  |  |
| Sodium hydroxide |  | 1310-73-2 | 1 | 0 | D002 | 0 | 1 | 3 |
| ### Detergent, automatic dishmachine |  |  |  |  |  |  |  |  |
| Sodium Hydroxide |  | 1310-73-2 | 1 | 0 | D002 | 0 | 1 | 3 |
| Sodium tripolyphosphate |  | 7758-29-4 | DUST | 0 | 0 | 0 | 1 | 2 |
| ### Detergent, automatic dishmachine |  |  |  |  |  |  |  |  |
| Sodium hydroxide |  | 1310-73-2 | 1 | 0 | D002 | 0 | 1 | 3 |
| Sodium dichloroisocyanurate dihydrate |  | 51580-86-0 | 0 | 0 | 0 | 0 | 1 | 1 |
| Detergent, destaining |  |  |  |  |  |  |  |  |
| Sodium Metasilicate |  | 6834-92-01 | 1 | 0 | D002 | 0 | 1 | 3 |
| Sodium perborate monohydrate |  | 7632-04-41 | 0 | 0 | D001 | 0 | 1 | 2 |
| ### Detergent, destaining |  |  |  |  |  |  |  |  |
| Sodium metasilicate |  | 6834-92-0 | 1 | 0 | D002 | 0 | 1 | 3 |
| Sodium Perborate |  |  | 0 | 0 | D001 | 0 | 1 | 2 |
| ### Detergent, dish, hand, liquid |  |  |  |  |  |  |  |  |
| Sodium lauryl ether sulfate |  | 9004-82-4 | 0 | 0 | 0 | 0 | 0 | 0 |
| ### Detergent, disinfectant |  |  |  |  |  |  |  |  |
| Alkyldimethylbenzyl ammonium chlorides |  | 68424-85-1 | 0 | 0 | PEST | 0 | 1 | 2 |
| Ethanolamine (MEA) |  | 141-43-5 | 1 | 0 | D002 | 0 | 0 | 2 |
| ### Detergent, disinfectant, iodophor |  |  |  |  |  |  |  |  |
| Complexed Iodine |  | 7553-56-2 | 1 | 0 | 0 | 0 | 1 | 2 |
| Phosphoric Acid |  | 7664-38-2 | 1 | 0 | D002 | 0 | 1 | 3 |
| Butoxyethanol |  | 111-76-2 | 1 | 0 | D002 | 0 | 1 | 3 |
| ### Detergent, hand, dish, liquid |  |  |  |  |  |  |  |  |
| Nonionic surfactant |  | confidential |  |  |  |  |  | * |
| Anionic Surfactant |  | confidential |  |  |  |  |  | * |
| ### Detergent, dish, hand, solid |  |  |  |  |  |  |  |  |
| none |  |  | 0 | 0 | 0 | 0 |  | 0 |
| ### Detergent, disinfectant, quaternary |  |  |  |  |  |  |  |  |
| Alkyldimethylbenzyl ammonium chlorides |  | 68424-85-1 | 1 | 0 | PEST | 0 | 1 | 2 |
| Ethanolamine (MEA) |  | 141-43-5 | 1 | 0 | D002 | 0 | 1 | 3 |
| ### Detergent, high alkaline, solid |  |  |  |  |  |  |  |  |
| Sodium Metasilicate |  | 6834-92-0 | 1 | 0 | D002 | 0 | 1 | 3 |
| Detergent, pot cleaning | Pot Shot (TM) |  |  |  |  |  |  |  |
| Ethanol |  | 64-17-51 | 1 | 0 | D001 | 0 | 1 | 3 |
| ### Detergent, presoak |  |  |  |  |  |  |  |  |
| Sodium Metasilicate |  | 6934-92-0 | 1 | 0 | D002 | 0 | 1 | 3 |
| ### Detergent, sanitizer |  |  |  |  |  |  |  |  |
| Sodium Dichloroisocyanurate Dihydrate |  | 51580-86-0 | 0 | 0 | 0 | 0 | 1 | 1 |
| Trisodium Phosphate |  | 7601-54-9 |  |  |  |  |  |  |
| Sodium Carbonate |  | 497-19-8 | DUST | 0 | 0 | 0 | 1 | 1 |
| ### Detergent, solid, automatic dishmachine |  |  |  |  |  |  |  |  |
| Sodium Hydroxide |  | 1310-73-2 | 1 | 0 | D002 | 0 | 1 | 3 |
| Sodium dichloroisocyanurate Dihydrate |  | 51580-86-0 | 0 | 0 | 0 | 0 | 1 | 1 |
| ### Detergent, solid, pot and pan |  |  |  |  |  |  |  |  |
| Nonylphenoxypoly (ethyleneoxy) ethanol |  | 9016-45-9 | 0 | 0 | 0 | 0 | 1 | 1 |
| Polyethylene Glycol |  | 25322-68-3 | 0 | 0 | 0 | 0 | 1 | 1 |
| ### Floor Cleaner |  |  |  |  |  |  |  |  |
| Nonylphenoxy Plyethoxy Glycol |  | 9016-45-9 | 0 | 0 | 0 | 0 | 1 | 1 |
| D-limonene |  | 138-86-3 | 0 | 0 | D001 | 0 | 1 | 1 |
| ### Floor cleaner, ammoniated, concentrate |  |  |  |  |  |  |  |  |
| Potassium hydroxide |  | 1310-58-3 | 1 | 0 | D002 | 0 | 1 | 3 |
| Ammonium Hydroxide |  | 1336-21-6 | 1 | 0 | D002 | 0 | 1 | 3 |
| ### Floor cleaner, detergent |  |  |  |  |  |  |  |  |
| Sodium Metasilicate |  | 6834-92-0 | 1 | 0 | D002 | 0 | 1 | 3 |
| Sodium Carbonate |  | 497-19-8 | 0 | 0 | 0 | 0 | 1 | 1 |
| Trisodium Phosphate |  | 7601-54-9 | 0 | 0 | 0 | 0 | 1 | 1 |
| ### Floor cleaner, aqueous acidic/solvent |  |  |  |  |  |  |  |  |
| Phosphoric Acid |  | 764-38-2 | 1 | 0 | D002 | 0 | 1 | 3 |
| Butoxyethanol |  | 111-76-2 | 1 | 0 | D002 | 0 | 1 | 3 |
| ### Flood cleaner, germicidal |  |  |  |  |  |  |  |  |
| Nonylphenoxypoly (Ethyleneoxy) Ethanol |  | 9016-45-9 | 0 | 0 | PEST | 0 | 1 | 1 |
| Quaternary Ammonium Chlorides |  | 68391-01-5 | 0 | 0 | PEST | 0 | 1 | 2 |
| ### Floor stripper |  |  |  |  |  |  |  |  |
| Ammonium Hydroxide |  | 1336-21-6 | 1 | 0 | D002 | 0 | 1 | 3 |
| Sodium Metasilicate |  | 6834-92-0 | 1 | 0 | D002 | 0 | 1 | 3 |
| Ethanolamine (MEA) |  | 141-43-5 | 1 | 0 | D002 | 0 | 1 | 3 |
| Butoxyethanol |  | 111-76-2 | 1 | 0 | D002 | 0 | 1 | 3 |
| ### Glass Cleaner |  |  |  |  |  |  |  |  |
| Alcohol isopropyl |  | 67-63-0 | 1 | 0 | D001 | 0 |  | 2 |
| Butoxyethanol |  | 111-76-2 | 1 | 0 | D002 | 0 |  | 2 |
| ### Oven Cleaner |  |  |  |  |  |  |  |  |
| Sodium Hydroxide |  | 1310-73-2 | 1 | 0 | D002 | 0 | 1 | 3 |
| ### Oven, cleaner, degreaser |  |  |  |  |  |  |  |  |
| Sodium hydroxide |  | 1310-73-2 | 1 | 0 | D002 | 0 | 1 | 3 |
| Ethanolamine (MEA) |  | 141-43-5 | 1 | 0 | D002 | 0 | 1 | 3 |
| ### Oven Cleaner/Degreaser |  |  |  |  |  |  |  |  |
| Sodium hydroxide |  | 1310-73-2 | 1 | 0 | D002 | 0 | 1 | 3 |
| Ethanolamine (MEA) |  | 141-43-5 | 1 | 0 | D002 | 0 | 1 | 3 |
| ### Oven/grill, cleaner |  |  |  |  |  |  |  |  |
| Sodium Hydroxide |  | 1310-73-21 | 1 | 0 | D002 | 0 | 1 | 3 |
| Potassium Hydroxide |  | 1310-58-31 | 1 | 0 | D002 | 0 | 1 | 3 |
| Tetrapotassium Pyrophosphate |  | 7320-34-51 | 0 | 0 | 0 | 0 | 1 | 1 |
| ### Pesticide, aerosol | Bolt (TM) |  |  |  |  |  |  |  |
| Heavy Alkylate Naphtha (as petroleum distillates) |  | 64741-65-7 | 1 | 0 | D001 | 0 | 1 | 3 |
| Tetramethrin |  | 7696-12-0 | 0 | 0 | PEST | 0 | 1 | 2 |
| d-(Cis, Trans) Phenothrin |  | 26002-80-2 | 0 | 0 | PEST | 0 | 1 | 2 |
| Liquefied Petroleum Gas |  | 68476-85-7 | 1 | 0 | D001 | 0 | 1 | 3 |
| # Pesticide, flying insect, aerosol | Down & Out (TM) |  |  |  |  |  |  |  |
| Propane |  | 74-98-6 | 1 | 0 | D001 | 0 | 1 | 3 |
| Isobutane |  | 75-28-5 | 1 | 0 | D001 | 0 | 1 | 3 |
| N-Butane |  | 106-97-8 | 1 | 0 | D001 | 0 | 1 | 3 |
| Tetramethrin |  | 7696-12-0 | 1 | 0 | PEST | 0 | 1 | 3 |
| Petroleum Distillate |  | 64742-48-9 | 1 | 0 | D001 | 0 | 1 | 3 |
| ### Presoak |  |  |  |  |  |  |  |  |
| Potassium Hydroxide |  | 1310-58-3 | 1 | 0 | D002 | 0 | 1 | 3 |
| Sodium Hypochlorite |  | 7681-52-9 | 1 | 0 | D003 | 0 | 1 | 3 |
| Sodium Tripolyphosphate |  | 7758-29-4 | 0 | 0 | 0 | 0 | 1 | 1 |
| Rinse additive | Rinse Aid (TM) |  |  |  |  |  |  |  |
| Sodium 2-Ethyl Hexyl Sulfate |  | 126-92-1 | 0 | 0 | 0 | 0 | 1 | 1 |
| Ethoxylated Propoxylated Linear Alcohols |  | 69013-18-9 | 0 | 0 | 0 | 0 | 1 | 1 |
| ### Rinse additive, dishmachine |  |  |  |  |  |  |  |  |
| Polyoxypropylene-Polyoxyethylene Black Copolymer |  | 9003-11-61 | 0 | 0 | 0 | 0 |  | 0 |
| ### Rinse agent |  |  |  |  |  |  |  |  |
| none |  |  | 0 | 0 | 0 | 0 | 0 | 0 |
| ### Sanitizer |  |  |  |  |  |  |  |  |
| Alkyldimethylbenzyl Ammonium Chlorides |  | 68424-85-1 | 0 | 0 | PEST | 0 | 1 | 2 |
| ### Sanitizer, iodine |  |  |  |  |  |  |  |  |
| Phosphoric Acid |  | 764-38-2 | 1 | 0 | D002 | 0 | 1 | 3 |
| Octylphenoxy Polyoxyethylene Iodine Complex |  | 11096-42-7 | 0 | 0 | 0 | 0 | 1 | 2 |
| Alkyl Polyoxyethylene Polyoxypropylene Iodine Complex |  | 68439-47-4 | 0 | 0 | PEST | 0 | 1 | 2 |
| Alcohols C-11-15 Ethoxgeated with iodine* |  |  |  |  |  |  |  |  |
| ### Sanitizer, iodine-based |  |  |  |  |  |  |  |  |
| Ethanol-iodine Complex |  | 11096-42-7 | 0 | 0 | D001 | 0 | 1 | 2 |
| ### Sanitizer, quaternary ammonium |  |  |  |  |  |  |  |  |
| Alkyl dimethyl benzyl ammonium chloride |  | 68242-85-1 | 0 | 0 | PEST | 0 | 1 | 2 |
| ### Scouring cleanser with bleach | Ajax (TM) |  |  |  |  |  |  |  |
| Silica |  | 7664-41-7 | 1 | 0 | 0 | 0 | 1 | 2 |
| Sodium hypochlorite hypochlorus acid |  | 7681-52-9 | 0 | 0 | D003 | 0 | 1 | 1 |
| ### Silverware presoak | Silver Sparkle (TM) |  |  |  |  |  |  |  |
| Sodium Metasilicate |  | 6834-92-01 | 1 | 0 | D002 | 0 | 1 | 3 |
| # Silverware Pre-Soak |  |  |  |  |  |  |  |  |
| Sodium carbonate |  | 497-19-8 | DUST | 0 | 0 | 0 | 1 | 1 |
|  |  |  |  |  |  |  |  | 0 |
| ### Silverware presoak, enzymatic |  |  |  |  |  |  |  |  |
| Sodium Carbonate |  | 497-19-8 | 0 | 0 | 0 | 0 |  | 0 |
| ### Stainless Steel Cleaner |  |  |  |  |  |  |  |  |
| Petroleum Distillate |  |  | 1 | 0 | D001 | 0 |  | 2 |
| ### Water Softener Salt Pellets |  |  |  |  |  |  |  |  |
| Sodium Chloride |  | 7647-14-5 | 0 | 0 | 0 | 0 | 1 | 1 |

PHARMACY Prioritization Tool

| CAS # | HAZARDOUS CHEMICAL; ## GENERIC NAME | RCRA CODE | P LIST | RCRA | OSHA | PBT | VOLUME | TOTAL |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 645-05-6 | ALTRETAMINE |  |  |  | 1 |  |  | 1 |
| 125-84-8 | AMINOGLUTETHIMIDE |  |  |  | 1 |  |  | 1 |
| 446-86-6 | AZATHIOPRINE |  |  |  | 1 |  |  | 1 |
| 7440-39-3 | BARIUM | D005 |  | 1 | 1 |  |  | 2 |
| 9041-93-4 | BLEOMYCIN |  |  |  | 1 |  | 1 | 2 |
| 55-98-1 | BUSULFAN |  |  |  | 1 |  |  | 1 |
| 7440-43-9 | CADMIUM | D006 |  | 1 |  | 1 |  | 2 |
| 41575-94-4 | CARBOPLATIN |  |  |  | 1 |  | 1 | 2 |
| 154-93-8 | CARMUSTINE |  |  |  | 1 |  | 1 | 2 |
| 305-03-3 | CHLORAMBUCIL | U035 |  | 1 | 1 |  |  | 2 |
| 56-75-7 | CHLORAMPHENICOL |  |  |  | 1 |  |  | 1 |
| 67-66-3 | CHLOROFORM | UO44/DO22 |  | 1 |  | 1 |  | 2 |
| 569-57-3 | CHLOROTRIANISENE |  |  |  | 1 |  |  | 1 |
| 54749-90-5 | CHLOROZOTOCIN |  |  |  | 1 |  |  | 1 |
| 7440-47-3 | CHROMIUM | D007 |  | 1 |  | 1 |  | 2 |
| 15663-27-1 | CISPLATIN |  |  |  | 1 |  |  | 1 |
| 8001-58-9 | CREOSOTE | U051 |  | 1 |  |  |  | 1 |
| 1319-77-3 | CRESOL (CRESYLIC ACID) | U052/D026 |  | 1 | 1 |  |  | 2 |
| 50-18-0 | CYCLOPHOSPHAMIDE | U058 |  | 1 | 1 |  | 1 | 3 |
| 59865-13-3 | CYCLOSPORIN |  |  |  | 1 |  |  | 1 |
| 147-94-4 | CYTARABINE |  |  |  | 1 |  | 1 | 2 |
| 4342 03-4 | DACARBAZINE |  |  |  | 1 |  | 1 | 2 |
| 50-76-0 | DACTINOMYCIN |  |  |  | 1 |  | 1 | 2 |
| 20830-81-3 | DAUNORUBICIN | U059 |  | 1 | 1 |  | 1 | 3 |
| 75-71-8 | DICHLORODIFLUOROMETHANE | U075 |  | 1 | 1 |  |  | 2 |
| 117-81-7 | Di (ethylhexyl) phthalate (component of PVC IV Bags & Other soft plastic equipment) | U028 |  | 1 | 1 | 1 | 1 | 4 |
| 56-53-1 | DIETHYLSTILBESTROL | U089 |  | 1 | 1 |  |  | 2 |
| 23214-92-8 | DOXORUBICIN |  |  |  | 1 |  | 1 | 2 |
| 51-43-4 | EPINEPHRINE | P042 | 1 | 1 |  |  | 1 | 3 |
| 50-28-2 | ESTRADIOL |  |  |  | 1 |  |  | 1 |
| 2998-57-4 | ESTRAMUSTINE |  |  |  | 1 |  |  | 1 |
| 57-63-6 | ETHINYL ESTRADIOL |  |  |  | 1 |  |  | 1 |
| 33419-42-0 | ETOPOSIDE |  |  |  | 1 |  | 1 | 2 |
| 50-91-9 | FLOXURIDINE |  |  |  | 1 |  | 1 | 2 |
| 51-21-8 | FLUOROURACIL |  |  |  | 1 |  | 1 | 2 |
| 13311-84-7 | FLUTAMIDE |  |  |  | 1 |  |  | 1 |
| 82410-32-0 | GANCICLOVIR |  |  |  | 1 |  |  | 1 |
| 70-30-4 | HEXACHLOROPHENE | U132 |  | 1 | 1 |  |  | 2 |
| 127-07-1 | HYDROXYUREA |  |  |  | 1 |  |  | 1 |
| 58957-92-9 | IDARUBICIN |  |  |  | 1 |  |  | 1 |
| 3778-73-2 | IFOSFAMIDE |  |  |  | 1 |  | 1 | 2 |
| 76543-88-9 | INTERFERON ALFA-2A |  |  |  | 1 |  |  | 1 |
| 99210-65-8 | INTERFERON ALFA-2B |  |  |  | 1 |  |  | 1 |
| 4759-48-2 | ISOTRETINOIN |  |  |  | 1 |  |  | 1 |
| 9015-68-3 | L-ASPARAGINASE |  |  |  | 1 |  | 1 | 2 |
| 53714-56-0 | LEUPROLIDE |  |  |  | 1 |  |  | 1 |
| 14764-73-4 | LEVAMISOLE |  |  |  | 1 |  |  | 1 |
| 58-89-9 | LINDANE | U129/D013 |  | 1 |  |  | 1 | 2 |
| 13010-47-4 | LOMUSTINE |  |  |  | 1 |  |  | 1 |
| 108-39-4 | m-CRESOL | D024 |  | 1 | 1 |  |  | 2 |
| 51-75-2 | MECHLORETHAMINE |  |  |  | 1 |  | 1 | 2 |
| 520-85-4 | MEDROXYPROGESTERONE |  |  |  | 1 |  |  | 1 |
| 595-33-5 | MEGESTROL ACETATE |  |  |  | 1 |  |  | 1 |
| 148-82-3 | MELPHALAN | U150 |  | 1 | 1 |  |  | 2 |
| 50-44-2 | MERCAPTOPURINE |  |  |  | 1 |  |  | 1 |
| 7439-97-6 | MERCURY | U151/D009 |  | 1 | 1 | 1 | 1 | 4 |
| 59-05-2 | METHOTREXATE |  |  |  | 1 |  | 1 | 2 |
| 50-07-7 | MITOMYCIN C | U010 |  | 1 | 1 |  |  | 2 |
| 53-19-0 | MITOTANE |  |  |  | 1 |  |  | 1 |
| 65271-80-9 | MITOXANTRONE |  |  |  | 1 |  | 1 | 2 |
| 76932-56-4 | NAFARELIN |  |  |  | 1 |  |  | 1 |
| 54-11-5 | NICOTINE | P075 | 1 | 1 | 1 |  | 1 | 4 |
| 55-63-0 | NITROGLYCERIN | P081 | 1 | 1 |  |  | 1 | 3 |
| 95-48-7 | o-CRESOL | D023 |  | 1 | 1 |  |  | 2 |
| 123-63-7 | PARALDEHYDE | U182 |  | 1 | 1 |  |  | 2 |
| 106-44-5 | p-CRESOL | D025 |  | 1 | 1 |  |  | 2 |
| 62-44-2 | PHENACETIN | U187 |  | 1 |  |  |  | 1 |
| 108-95-2 | PHENOL | U188 |  | 1 | 1 | 1 |  | 3 |
| 62-38-4 | PHENYLMERCURIC ACETATE | P092/D009 | 1 | 1 | 1 | 1 |  | 4 |
| 57-64-7 | PHYSOSTIGMINE SALICYLATE | P188 | 1 | 1 |  |  |  | 2 |
| 57-47-6 | PHYSOSTIGMINE | P204 | 1 | 1 |  |  |  | 2 |
| 54-91-1 | PIPOBROMAN |  |  |  | 1 |  |  | 1 |
| 18378-89-7 | PLICAMYCIN |  |  |  | 1 |  |  | 1 |
| 671-16-9 | PROCARBAZINE |  |  |  | 1 |  |  | 1 |
| 50-55-5 | RESERPINE | U200 |  | 1 |  |  |  | 1 |
| 108-46-3 | RESORCINOL | U201 |  | 1 |  |  |  | 1 |
| 36791-04-5 | RIBAVIRIN |  |  |  | 1 |  |  | 1 |
| 81-07-2 | SACCHARIN | U202 |  | 1 |  |  |  | 1 |
| 128-44-9 | SACCHARIN SODIUM | U202 |  | 1 |  |  |  | 1 |
| 7782-49-2 | SELENIUM | D010 |  | 1 | 1 | 1 |  | 3 |
| 7488-56-4 | SELENIUM SULFIDE | U205 |  | 1 | 1 | 1 | 1 | 4 |
| 7440-22-4 | SILVER | D011 |  | 1 | 1 |  |  | 2 |
| 7632-00-0 | SODIUM NITRITE | D002 |  | 1 | 1 |  |  | 2 |
| 18883-66-4 | STREPTOZOCIN | U206 |  | 1 | 1 |  |  | 2 |
| 57-24-9 | STRYCHNINE | P108 | 1 | 1 | 1 |  |  | 3 |
| 10540-29-1 | TAMOXIFEN |  |  |  | 1 |  |  | 1 |
| 968-93-4 | TESTOLACTONE |  |  |  | 1 |  |  | 1 |
| 54-64-8 | THIMEROSAL (CONTAINS MERCURY) | D009 |  | 1 |  | 1 | 1 | 3 |
| 154-42-7 | THIOGUANINE |  |  |  | 1 |  |  | 1 |
| 52-24-4 | THIOTEPA |  |  |  | 1 |  |  | 1 |
| 79-01-6 | TRICHLOROETHYLENE | U228 |  | 1 | 1 |  |  | 2 |
| 75-69-4 | TRICHLOROMONOFLUOROMETHANE | U121 |  | 1 |  |  |  | 1 |
| 66-75-1 | URACIL MUSTARD | U237 |  | 1 | 1 |  |  | 2 |
| 5536-17-4 | VIDARABINE |  |  |  | 1 |  |  | 1 |
| 865-21-4 | VINBLASTINE |  |  |  | 1 |  | 1 | 2 |
| 57-22-7 | VINCRISTINE |  |  |  | 1 |  | 1 | 2 |
| 81-81-2 | WARFARIN POTASSIUM (>.3%) | P001 | 1 | 1 | 1 |  | 1 | 4 |
| 81-81-2 | WARFARIN POTASSIUM (<0.3%) | U248 |  | 1 | 1 |  | 1 | 3 |
| 129-06-6 | WARFARIN SODIUM (<0.3%) | U248 |  | 1 | 1 |  | 1 | 3 |
| 129-06-6 | WARFARIN SODIUM (>0.3%) | P001 | 1 | 1 | 1 |  | 1 | 4 |
| 30516-87-1 | ZIDOVUDINE |  |  |  | 1 |  |  | 1 |
| 7440-66-6 | ZINC |  |  |  |  | 1 |  | 1 |
| 557-34-6 | ZINC ACETATE |  |  |  |  | 1 |  | 1 |
| 04468-02-4 | ZINC GLUCONATE |  |  |  |  | 1 |  | 1 |
| 1314-13-2 | ZINC OXIDE |  |  |  |  | 1 |  | 1 |
| 7733-02-0 | ZINC SULFATE |  |  |  |  | 1 |  | 1 |
| 557-08-4 | ZINC UNDECYLENATE |  |  |  |  | 1 |  | 1 |
| CAS # | ### COMPOUNDING CHEMICALS | RCRA CODE | P LIST | RCRA | OSHA | PBT | VOLUME | TOTAL |
| 64-19-7 | ACETIC ACID | D001/D002 |  | 1 | 1 |  | 1 | 3 |
| 67-64-1 | ACETONE | U002/D001 |  | 1 | 1 |  | 1 | 3 |
| 64-17-5 | ALCOHOL (ETHYL) | D001 |  | 1 | 1 |  | 1 | 3 |
| 10-982-7 | ALCOHOL (ISOPROPYL) | D001 |  | 1 | 1 |  | 1 | 3 |
| 39-939-6 | BENZOIN (TINCTURE) | D001 |  | 1 |  |  |  | 1 |
| 56-23-5 | CARBON TETRACHLORIDE | U211 |  | 1 | 1 |  |  | 2 |
| 8007-45-2 | COAL TAR | D001 |  | 1 |  |  |  | 1 |
| 60-29-7 | ETHER | D001 |  | 1 | 1 |  |  | 2 |
| 50-00-0 | FORMALDEHYDE | U122/D001 |  | 1 | 1 |  |  | 2 |
| 64-18-6 | FORMIC ACID | U123/D002 |  | 1 | 1 |  |  | 2 |
|  | GREEN SOAP TINCTURE | D001 |  | 1 |  |  |  | 1 |
| 1647-01-0 | HYDROCHLORIC ACID | D002 |  | 1 | 1 |  |  | 2 |
| 7553-56-2 | IODINE | D001/D002 |  | 1 | 1 |  |  | 2 |
| 21908-53-2 | MERCURIC OXIDE | D009 |  | 1 |  | 1 |  | 2 |
| 129-16-8 | MERCUROCHROME (MERBROMIN) | D009 |  | 1 |  | 1 |  | 2 |
| 67-56-1 | METHANOL | U154/D001 |  | 1 | 1 |  |  | 2 |
| 7697-37-2 | NITRIC ACID | D002 |  | 1 | 1 |  |  | 2 |
| 144-62-7 | OXALIC ACID | D002 |  | 1 | 1 |  |  | 2 |
| 7664-38-2 | PHOSPHORIC ACID | D002 |  | 1 | 1 |  |  | 2 |
| 518-28-5 | PODOPHYLLUM RESIN | D002 |  | 1 |  |  |  | 1 |
| 1310-58-3 | POTASSIUM HYDROXIDE | D002 |  | 1 |  |  |  | 1 |
| 7722-64-7 | POTASSIUM PERMANGANATE | D001 |  | 1 |  |  |  | 1 |
| 7761-88-3 | SILVER NITRATE | D011/D001 |  | 1 |  |  | 1 | 2 |
| 22199-03-2 | SILVER SULFADIAZINE | D011 |  | 1 |  |  | 1 | 2 |
| 1310-73-2 | SODIUM HYDROXIDE | D002 |  | 1 | 1 |  |  | 2 |
| 7681-52-9 | SODIUM HYPOCHLORITE | D002 |  | 1 | 1 |  |  | 2 |
| 7664-93-9 | SULFURIC ACID | D002 |  | 1 | 1 |  |  | 2 |
| 127-18-4 | TETRACHLOROETHYLENE | U210/D039 |  | 1 | 1 |  |  | 2 |
| 76-03-9 | TRICHLOROACETIC ACID | D002 |  | 1 |  |  |  | 1 |
| 79-01-6 | TRICHLOROETHYLENE | U228/D040 |  | 1 | 1 |  |  | 2 |

RADIOLOGY Prioritization Tool

| CHEMICAL NAME & INGREDIENTS | COMMON/TRADE NAME OR USE | CAS# | OSHA | PBT | RCRA | P? | VOL | TOTAL |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ### Sodium Hypochlorite | Bleach | 7681-52-9 | 1 | 0 | D003 | 0 | 1 | 3 |
| ### Isopropyl Alcohol | IPA, rubbing alcohol | 67-63-0 | 1 | 0 | D001 | 0 | 0 | 2 |
| ### Developer |  |  |  |  |  |  | 0.5 | 0.5 |
| diethylene glycol |  |  | 0 | 0 | 0 | 0 | 1 | 1 |
| acetic acid |  | 64-19-7 | 1 | 0 | D002 | 0 | 1 | 3 |
| ### Chromic Acid Cleaner | cleaner |  |  |  |  |  |  |  |
| chromium |  |  | 1 | 1 | D007 | 0 |  | 3 |
| ### Glutaraldehyde |  | 111-30-8 | 0 | 0 | PEST | 0 | 0.5 | 2.5 |
| ### Developer |  |  |  |  |  |  |  |  |
| Potassium Hydroxide |  | 1310-58-3 | 1 | 0 | D002 | 0 | 1 | 3 |
| Developer Starter |  |  |  |  |  |  |  |  |
| Acetic Acid |  | 64-19-7 | 1 | 0 | D002 | 0 | 0 | 2 |
| ### Fixer Replenisher |  |  |  |  |  |  |  |  |
| Acetic Acid |  | 64-19-7 | 1 | 0 | D002 | 0 | 0.5 | 2.5 |
| ### Fixer Working Strength |  |  |  |  |  |  |  |  |
| Aqueous Acidic Solution* |  |  |  |  |  |  | 1 | *1 |
| ### Photodrome - Lab Pak II |  |  |  |  |  |  |  |  |
| 1-Bromo-3-chloro-5 Dimethylhydantoin |  | 126-06-7 | 0 | 0 | PEST | 0 | 0 | 1 |
| Spent Fixer (contains acetic acid and silver) |  |  | 1 | 0 | D002, D011 | 0 | 1 | 3 |
| ### Toner |  |  |  |  |  |  |  |  |
| selenium |  | 7782-49-2 | 1 | 1 | D010 | 0 | 1 | 4 |
| Di (ethylhexyl)phthalate | (found in soft plastic, PVC items, e.g. IV bags, tubing, etc.) |  | 1 | 1 | U028 | 0 | 1 | 4 |

.

STERILE PROCESSING Prioritization Tool

| CHEMICAL NAME & INGREDIENTS | COMMON/TRADE NAME OR USE | #### CAS # | OSHA | #### PBT | RCRA | #### P? |  | #### VOL | TOTAL |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ##### Acetone | Solvent, nail polish remover | 67-64-1 | 1 | 0 | F003/ U002 | 0 |  | 0 | 1 |
| Phenolic Germicidal Soap | Vesphene |  |  |  |  |  |  |  |  |
| sodium o-phenylphenate |  | 90-43-7 | 0 | 0 | PEST | 0 |  | 0.5 | 1.5 |
| sodium o-benzyl-p-chlorophenate |  | 120-32-1 | 0 | 0 | PEST | 0 |  | 0.5 | 1.5 |
| sodium p-tertiary amylphenate |  | 80-46-6 | 0 | 0 | PEST | 0 |  | 0.5 | 1.5 |
| Enzymatic Cleaner |  |  |  |  |  |  |  |  |  |
| Diethanolamine |  | 111-42-2 | 1 | 0 | D002 | 0 |  | 1 | 3 |
| Bacterial Protease* |  |  | 0 | 0 | * | 0 |  | 1 | 1 |
| Surgical Instrument Cleaner | Blitz |  |  |  |  |  |  |  |  |
| alkylate naptha |  | 64741-65-7 | 1 | 0 | D001 | 0 |  | 0 | 2 |
| paraffinic distillates |  | 64742-65-0 | 1 | 0 | D001 | 0 |  | 0 | 2 |
| isobutane/butane |  | 75-28-5, 74-98-6 | 1 | 0 | D001 | 0 |  | 0 | 2 |
| Castille Soap | castille soap |  |  |  |  |  |  |  |  |
| sodium soap of mixed fatty acids |  |  | 0 | 0 | 0 | 0 |  | 0 | 0 |
| Di(ethyl hexyl) phthalate (DEHP) | PVC IV bags, tubing , other soft plastic equipment |  | 1 | 1 | 1 | 0 |  | 1 | 4 |
| Glutaraldehyde 2% | Cidex | 111-30-8 | 0 | 0 | PEST | 0 |  | 1 | 2 |
| Ethylene Oxide | Ethylene oxide | 75-21-8 | 1 | 0 | U115 | 0 |  | 1 | 3 |
| Petroleum Hydrocarbon | lubricating oil |  | 0 | 0 | 0 | 0 |  | 0 | 0 |
| 3% Hydrogen Peroxide | Hydrogen Peroxide | 7722-84-1 | 1 | 0 | 0 | 0 |  | 0 | 1 |
| Low Foam Detergent |  |  |  |  |  |  |  |  |  |
| Nonionic Surfactant* |  |  |  |  |  |  |  |  |  |
| Isopropyl Alcohol | Rubbing Alcohol, IPA | 67-63-0 | 1 | 0 | D001 | 0 |  | 0 | 2 |
| Cleansing Polish | Liberty |  |  |  |  |  |  |  |  |
| Amorphous silica |  | 14808-60-7 | 1 | 0 | 0 | 0 |  | 0 | 1 |
| Ammonium hydroxide |  | 1336-21-6 | 0 | 0 | D002 | 0 |  | 0 | 1 |
| Isopropanol |  | 67-63-0 | 1 | 0 | D001 | 0 |  | 0 | 2 |
| Aluminum sulfate |  | 10043-01-3 | 0 | 0 | 0 | 0 |  | 0 | 0 |
| Citric acid |  | 77-92-9 | 0 | 0 | 0 | 0 |  | 0 | 0 |
| ##### Scale Cleaner | Scale-gon Klenz |  |  |  |  |  |  |  |  |
| ##### Phosphoric Acid |  | 7664-38-2 | 1 | 0 | D002 | 0 |  | 0.5 | 2.5 |
| Liquid Cream Cleanser | ##### Spar Crème |  |  |  |  |  |  |  |  |
| Citric acid |  | 77-92-9 | 0 | 0 | 0 | 0 |  | 0 | 0 |
| silica, crystalline quartz |  | 14808-60-7 | 1 | 0 | 0 | 0 |  | 0 | 1 |
| Surgistain |  |  |  |  |  |  |  |  |  |
| Isopropanol |  | 67-63-0 | 1 | 0 | D001 | 0 |  | 0 | 2 |
| Phosphoric Acid |  | 7664-38-2 | 1 | 0 | D002 | 0 |  | 0 | 2 |
| Floor cleaner |  |  |  |  |  |  |  |  |  |
| Trisodium Phosphate |  | 10101-89-0 | 0 | 0 | 0 | 0 |  | 0 | 0 |
| Wexcide Germicidal Detergent |  |  |  |  |  |  |  |  |  |
| 2-phenylphenol |  | 90-43-7 | 0 | 0 | PEST | 0 |  | 1 | 2 |
| 0-benzyl-p-chlorophenol |  | 120-32-1 | 0 | 0 | PEST | 0 |  | 1 | 2 |
| Spot Remover, Alkaline | Xomed Cleaning Solution |  |  |  |  |  |  | 0 | 0 |
| ###### Carbon Tetrachloride | Spot Remover | 56-23-5 | 1 | 0 | U211/F002 | 0 | 0 |  | 2 |

SURGICAL SERVICES Prioritization Tool

| CHEMICAL NAME & INGREDIENTS | COMMON/TRADE NAME OR USE | CAS# | OSHA | PBT | RCRA | P? | VOL | TOTAL |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Aerosol Cleaner | Sheila Shine (TM) |  |  |  |  |  |  |  |
| 1,2,2-trichloroethane |  | 71-55-6 | 1 | 1 | F002 | 0 | 0 | 3 |
| xylene |  | 1330-20-7 | 1 | 0 | F003 | 0 | 0 | 2 |
| refined paraffins |  | 647-42-558 | 1 | 0 | D001 | 0 | 0 | 2 |
| methyl salicylate |  | 64-19-7 | 0 | 0 | 0 | 0 | 0 | 0 |
| # Aerozoin | Aerosol topical antiseptic |  |  |  |  |  |  |  |
| Tincture of Benzoin |  |  | 0 | 0 | D001 | 0 | 0 | 1 |
| Isopropanol |  |  | 1 | 0 | D001 | 0 | 0 | 2 |
| Propane isobutane |  |  | 1 | 0 | D001 | 0 | 0 | 2 |
| # All-purpose cleaner |  |  |  |  |  |  |  |  |
| 1-Methoxy 2-propanol |  | 107-98-2 | 1 | 0 | D001 | 0 | 0.5 | 2.5 |
| All-purpose cleaner with ammonia | Ajax (TM) |  |  |  |  |  |  |  |
| Ammonia |  | 7664-41-7 | 1 | 0 | D002 | 0 | 0.5 | 2.5 |
| # All-purpose cleaner with bleach | Comet (TM) |  |  |  |  |  |  |  |
| Silica |  |  | 1 | 0 | 0 | 0 | 0.5 | 1.5 |
| # Bath Cleaner |  |  |  |  |  |  |  |  |
| Citric Acid |  |  | 0 | 0 | 0 | 0 |  | 0 |
| # Anesthetic gases |  |  |  |  |  |  |  |  |
| # Benzoin Tincture | Benzoin Compund/topical disinfectant |  |  |  |  |  |  |  |
| Ethyl Alcohol | Ethanol | 64-17-5 | 1 | 0 | D001 | 0 | 0 | 2 |
| # Betadine solution |  |  |  |  |  |  |  |  |
| Povidone iodine USP |  | 25655-41-8 | 0 | 0 | 0 | 0 | 1 | 1 |
| # Glycerin |  | 56-81-5 | 0 | 0 | 0 | 0 | 1 | 1 |
| # Igepal |  |  | 0 | 0 | 0 | 0 | 0 | 0 |
| # Cleanser | Comet (TM) |  |  |  |  |  |  |  |
| Bleach (Sodium hypochlorite) |  | 7681-52-9 | 1 | 0 | D003 |  | 0.5 | 2.5 |
| # Colloidion Flexible | Colloidion |  | 0 | 0 | D001 | 0 | 0 | 1 |
| Cellulose |  |  | 0 | 0 | 0 | 0 | 0 | 0 |
| Di(ethyl hexyl) phthalate | PVC IV bags, tubing, other equip. |  | 1 | 1 | U028 | 0 | 1 | 4 |
| # Ethanol |  | 64-17-5 | 1 | 0 | D001 | 0 | 0 | 2 |
| # Ethyl Ether |  |  | 1 | 0 | D001 | 0 | 0 | 2 |
| # Camphor |  |  | 1 | 0 | 0 | 0 | 0 | 1 |
| # Defoamer |  |  |  |  |  |  |  | 0 |
| Propotol |  |  | 0 | 0 | 0 | 0 | 1 | 1 |
| Sodium Hydroxide |  |  | 1 | 0 | D002 | 0 | 1 | 3 |
| Glycerol |  |  | 1 | 0 | 0 | 0 | 1 | 2 |
| # Ferric subsulfate | Monsel's Solution/coagulant |  | 0 | 0 | 0 | 0 | 0 | 0 |
| # Floor Coating | Topline (TM) |  |  |  |  |  |  |  |
| Styrene-Acrylonitrile Polymer* |  | 9003-54-7 | 0 | 0 | 0 | 0 | 1 | 1 |
| Diethylene glycol monomethyl ether |  | 11-77-3 | * |  |  |  | 1 | 1 |
| Tributoxyethyl Phosphate |  | 78-51-3 | 1 | 0 | 0 | 0 | 1 | 2 |
| Zinc ammonia carbonate complex* |  | 38714-47-5 | * 0 | 0 | 0 | 0 | 1 | 1 |
| Maleic Anhydride-propylene polymer* |  | 25722-45-6 | * 0 | 0 | 0 | 0 | 1 | 1 |
| Polyethylene |  | 9002-88-4 | * 0 | 0 | 0 | 0 | 1 | 1 |
| Dibutyl Phthalate |  | 1310-45-8 | 1 | 1 | 1 | 0 | 1 | 4 |
| Formaldehyde Solution | Formalin |  |  |  |  |  |  |  |
| Methanol |  | 67-56-1 | 1 | 0 | F004/D001 | 0 | 1 | 3 |
| Formaldehyde |  | 50-00-0 | 1 | 0 | U122 | 0 | 1 | 3 |
| # Furniture Polish |  |  |  |  |  |  |  |  |
| Petroleum Distillates |  |  | 1 | 0 | D001 | 0 | 0 | 2 |
| Isobutanol Propane blend |  |  | 1 | 0 | D001 | 0 | 0 | 2 |
| # Germicidal Detergent | disinfecting detergent/"Quat" |  |  |  |  |  |  |  |
| n-n-bis (2-omega-hydropoly (oxyethylene) ethyl aklylamine |  | 61791-14-8 | 0 | 0 | PEST | 0 | 0 | 1 |
| quaternary ammonium compounds |  | 107-41-5 | 1 | 0 | PEST | 0 | 1 | 3 |
| hexylene glycol |  |  | 1 | 0 | PEST | 0 | 1 | 3 |
| # Isopropanol | Alcohol, rubbing |  | 1 | 0 | D001 | 0 | 1 | 3 |
| # Glacial Acetic acid | Acetic acid | 64-19-7 | 1 | 0 | D002 | 0 | 0 | 2 |
| Gum Remover (aerosol) | Misty (TM) |  |  |  |  |  |  |  |
| Isobutane/propane blend |  | 74-98-6, 75-28-5 | 1 | 0 | D001 | 0 | 0 | 2 |
| # Iodophor | Topical disinfectant |  | 1 | 0 | PEST | 0 | 0 | 2 |
| Isopropanol | IPA | 67-63-0 | 1 | 0 | D001 | 0 | 0.5 | 2.5 |
| # Lubricant/Lubricating Oil | WD40 |  |  |  |  |  |  |  |
| Aliphatic petroleum distillates |  |  | 1 | 0 | D001 | 0 | 0 | 2 |
| Hydrocarbon base oil | mineral oil | 8042-47-5 | 1 | 0 | 0 | 0 | 0 | 1 |
| # Mercury | Quicksilver |  | 1 | 1 | U151 | 0 | 0 | 3 |
| # Nail Polish Remover | Cutex (TM) |  |  |  |  |  |  |  |
| Acetone |  | 67-64-1 | 1 | 0 | F003 | 0 | 0 | 2 |
| Isopropanol |  | 67-63-0 | 1 | 0 | D001 | 0 | 0 | 2 |
| Butyl Acetate |  | 123-86-4 | 1 | 0 | D001 | 0 | 0 | 2 |
| Dibutyl phthalate |  |  | 1 | 1 | U069 | 0 | 0 | 3 |
| # One-step Cast | Scotch Cast (TM), splint kit |  |  |  |  |  |  |  |
| 4,4-diphenylmethane diisocyante-polypropylene glycol polymer |  | 65997-17-3 | * |  |  |  |  |  |
| foam, polyurethane ether transfer adhesive |  | 9048-57-1 | * |  |  |  |  |  |
| diphenylmethane diisocyante |  |  | 1 | 0 | D003 | 0 | 0 | 2 |
| dimorpholinodiethyl ether |  |  | * |  |  |  |  |  |
| 4 (2-1-methyl-2- (4-morpholinyl ethoxy)-ethyl morpholine |  |  | * |  |  |  |  |  |
| diphenylmethane diisocyante homopolymer |  |  | * |  |  |  |  |  |
| butylated hydroxy toluene |  | 128-37-0 | 1 | 0 | 0 | 0 | 0 | 1 |
| benzoyl chloride |  | 98-88-4 | 0 | 0 | D002 | 0 | 0 | 1 |
| # Phenol | disinfectant | 108-95-2 | 1 | 1 | D002 | 0 | 0.5 | 3.5 |
| # Porcelain & Tile cleaner | acid cleaner |  |  |  |  |  |  |  |
| Phosphoric acid |  | 7664-38-2 | 1 | 0 | D002 | 0 | 0.5 | 2.5 |
| nonylphenol ethoxylate |  | 64-02-8 | 0 | 0 | 0 | 0 | 0 | 0 |
| xanthum gum |  |  | 0 | 0 | 0 | 0 | 0 | 0 |
| # Silicone Foam |  |  |  |  |  |  |  |  |
| tetra-n-porphorthosilicate |  |  | 0 | 0 | 0 | 0 | 0 | 0 |
| # Sponge Brush | disinfectant scrub |  |  |  |  |  |  |  |
| 4-chloro-3,5 dimethyl phenol |  | 88-04-0 | 0 | 0 | 0 | 0 | 0.5 | 0.5 |
| # Spray Cleaner |  |  |  |  |  |  |  |  |
| Mineral spirits |  |  | 1 | 0 | D001 | 0 | 0 | 2 |
| Formaldehyde |  | 64741-65-7 | 1 | 0 | U122 | 0 | 0 | 2 |
| # Sterilant | Steris 20 (TM) |  |  |  |  |  |  |  |
| Peroxyacetic acid |  | 79-21-0 | 1 | 0 | D002 | 0 | 1 | 3 |
| # Strong Iodine Solution |  |  |  |  |  |  |  |  |
| Iodine |  | 7553-56-2 | 1 | 0 | PEST | 0 | 0 | 2 |
| Potassium Iodide |  | 7681-11-0 | 0 | 0 | PEST | 0 | 0 | 1 |
| # Surgistain |  |  |  |  |  |  |  |  |
| Phosphoric acid |  |  | 1 | 0 | D002 | 0 |  | 2 |
| 1-(2-methoxy-2-methoxy) 2-propanol |  | 7664-38-2 | 1 | 0 | D001 | 0 | 0 | 1 |
| Stainless Steel Cleaner & Polish | Aerosol stainless steel cleaner |  |  |  |  |  |  |  |
| White Mineral Oil |  | 50-0-0 | 1 | 0 | 0 | 0 | 0 | 1 |
| Isobutane |  | 8042-47-5 | 1 | 0 | D001 | 0 | 0 | 2 |
| Sorbitan Oleate |  | 74-98-6 | 0 | 0 | 0 | 0 | 0 | 0 |
| Ethanolamine |  | 141-43-5 | 1 | 0 | D002 | 0 | 0 | 2 |
| # Surgical Scrub |  |  |  |  |  |  |  |  |
| Anionic surfactant |  |  | * |  |  |  |  |  |
| Emollients |  |  | * |  |  |  |  |  |
| Chloroxylenol |  | 88-04-0 | 0 | 0 | PEST | 0 | 0 | 1 |
| # Surgical Solution | 3M Duraprep (TM) |  |  |  |  |  |  |  |
| Isopropanol |  | 67-63-0 | 1 | 0 | D001 | 0 | 0.5 | 2.5 |
| Ethyl Alcohol |  | 64-17-5 | 1 | 0 | D001 | 0 | 0.5 | 2.5 |
| Acrylate Polymer |  |  | * |  |  |  |  |  |
| Iodine |  | 7553-56-2 | 1 | 0 | ## PEST | 0 | 0.5 | 2.5 |
| Tannin(s), Gallotannin, Gallotannic acid, Digallic acid | Astringent | 1401-55-4 | 0 | 0 | 0 | 0 |  | 0 |
| # Trouble Shooter Cleaner | Aerosol cleaner |  |  |  |  |  |  |  |
| 2-Butoxyethanol |  | 111-76-2 | 1 | 0 | D002 | 0 | 0 | 2 |
| Ethanolamine |  | 141-43-5 | 1 | 0 | D002 | 0 | 0 | 2 |
| Propane/isobutane |  | 74-98-6, 75-28-5 | 1 | 0 | D001 | 0 | 0 | 2 |

“RCRA”, “P” or “P list” in Prioritization Tool (tables on p3-48).

“OSHA” in Prioritization Tool (tables on p 3-48).

“PBT” in Prioritization Tool (tables on p 3-48).

“Vol” or “Volume”  in Prioritization Tool (tables on p 3-48).

Acute Toxicity is defined as Oral LD50 of less than or equal to 500 mg/kg, Dermal LD50 of less than or equal to 2000 mg/kg, Inhalation LD50 of less than or equal to 0.05 mg/L or Eye Irritation that is corrosive (irreversible destruction of ocular tissue) or corneal involvement or irritation persisting for more than 21 days.  EPA Office of Pesticide Programs Label Review Manual Chapter 8: Precautionary Labeling.  www.epa.gov/oppfead1/labeling/lrm/chap-08.htm.

not done due to therapeutic nature.